Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : 14CE2003 |  | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : MECHANICS OF FLUIDS |  | # Max. marks : | # 100 |

# (for Biotech only)

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Two horizontal plates are placed 1 cm apart, and the space between them is filled with oil of viscosity 14 poises. Calculate the shear stress in oil, if upper plate is moved with a velocity of 2 m/s. | CO1 | 10 |
|  | b. | Capillary rise in a glass tube of 2.5 mm diameter is 1.8 cm when immersed in water. Calculate the surface tension of water in contact with air. | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Estimate the gauge pressure and absolute pressure at a point 4 m below the free surface of a liquid of sp. gravity 1.53, if the atmospheric pressure is 76 cm mercury. | CO1 | 10 |
|  | b. | A simple mercury manometer is connected to a pipe carrying oil of sp. gr 0.8. The pressure in the pipe is vacuum and the other end of manometer is open to atmosphere. If the difference in mercury level in two arms is 20 cm and height of oil in the left limb from center of pipe is 15 cm below, calculate the vacuum pressure in pipe. | CO1 | 10 |
| 3. | a. | Water is flowing through a tapered inclined pipe as shown in the diagram. Calculate the flow rate, if the pressure intensities at the two ends are 40 N/m2 and 30 N/m2, respectively. Diameter of the pipe at the two ends are 20 cm and 10 cm, respectively. Ignore any head loss in pipe. | CO2 | 15 |
|  | b. | Explain working principle of Pitot-tube (with mathematical expression or derivation) used for velocity measurement of a fluid. | CO2 | 5 |
| (OR) |  |  |  |  |
| 4. |  | An orifice meter with orifice diameter of 15 cm is inserted in a pipe of diameter 30 cm. Pressure difference on the two side of orifice meter is 30 cm mercury, as measured in differential manometer. Find the flowrate of oil (sp.gr. 0.9) when Cd is 0.64 | CO2 | 20 |
| 5. |  | The rate of flow of water through a horizontal pipe is 0.5 m3/s. The diameter of the pipe is suddenly contracted from 40 cm to 10 cm. The pressure intensity in smaller pipe is 10 N/cm2. Estimate (i) pressure in the large pipe, (ii) power loss due to contraction. Use Cc = 0.60 and f = 0.01 in Darcy’s equation if required. | CO2 | 20 |
| (OR) |  |  |  |  |
| 6. |  | A vertical pipe of 50 cm diameter carrying water is suddenly changes to 25 cm diameter. Pressure intensities5 m below and 5 m abovethe contraction/expansion point are 10 N/cm2 and 14 N/cm2. Calculate the direction and flow rate of water.Use Cc = 0.60 and f = 0.01 in Darcy’s equation if required. | CO2 | 20 |
| 7. |  | Water is flowing through a 100 m long pipe of 0.2 m diameter. The pressure at two ends of the pipe are 80 N/cm2 and 3 m Hg, respectively. Estimate the flow rate through the pipe, given f = 0.01 in Darcy’s equation. | CO3 | 20 |
| (OR) |  |  |  |  |
| 8. |  | A syphon of diameter 20 cm connects two reservoirs as difference in elevation of 20 m. The total length of syphon is 800 m and summit is 5 m above water level in upper reservoir. The pressure head at summit is 2.8 m water. Determine the length of syphon from upper reservoir to summit. Take f = 0.004 and atmospheric pressure as 10.3 m water. | CO3 | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | A tank has two identical orifices in one of its vertical side. The upper and lower orifices are located 4 m and 6 m below the water surface, respectively. If the value of cv for both the orifices is 0.9, find the point of intersection of the two jets. | CO3 | 15 |
|  | b. | Derive the expression to estimate theoretical discharge from a rectangular orifice of width ‘b’ having liquid height H1 and H2 from top and bottom end of orifice to surface. | CO3 | 5 |