Reg.No. ____________

End Semester Examination – Apr/May – 2018

| Code : 14CE2003 |  | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : MECHANICS OF FLUIDS |  | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Calculate the dynamic viscosity of an oil, which is used for lubrication between a square plate of size 0.8 m X 0.8 m and an inclined plane with angle of inclination 30o to the horizontal. The weight of the square plate is 300 N and it slides down the inclined plane with a uniform velocity of 0.3 m/s. The thickness of the oil film is 1.5 mm. | CO1 | 12 |
|  | b. | Derive the units for dynamic viscosity using Newton’s law of viscosity. | CO1 | 8 |
| (OR) |  |  |  |  |
| 2. | a. | Calculate the capillary effect in millimeters in a glass tube of 4 mm diameter, when immersed in (i) water, and (ii) mercury. The temperature of the liquid is 20oC and the values of the surface tension of water and mercury at 20oC in contact with air are 0.073575 N/m and 0.51 N/m respectively. The angle of contact for water is zero and that for mercury is 130o. Take density of water at 20oC as equal to 998 kg/m3. | CO1 | 15 |
|  | b. | Deduce the expression for pressure intensity inside a liquid droplet. | CO1 | 5 |
| 3. | a. | Deduce the expression for variation of pressure in a static fluid at rest. | CO1 | 7 |
|  | b. | A simple U-tube manometer containing mercury is connected to a pipe in which a fluid of specific gravity 0.8 and having vacuum pressure is flowing. The other end of the manometer is open to the atmosphere. Find the vacuum pressure in pipe, if the difference of mercury level in the two limbs is 40 cm and the height of fluid in the left from the centre of the pipe is 15 cm below. | CO2 | 13 |
| (OR) |  |  |  |  |
| 4. | a. | A rectangular plane surface 2 m wide and 3 m deep lies in water in such a way that its plane makes an angle of 30o with the free surface of water. Determine the total pressure and position of centre of pressure when the upper edge is 1.5 m below the free water surface. | CO1 | 13 |
|  | b. | Breakdown the steps to be followed in a arriving at the total resultant force on a curved surface with clear and precise equations and a neat schematic diagram. | CO1 | 7 |
| 5. | a. | The velocity components in a two-dimensional flow field for an incompressible fluid are as follows: , Obtain an expression for the stream function | CO2 | 14 |
|  | b. | Contrast the characteristics of a stream line, streak line and a path line with neat sketches. | CO1 | 6 |
| (OR) |  |  |  |  |
| 6. | a. | The velocity vector in a fluid flow is given as: +2tk Find the velocity and acceleration of a fluid particle at (2,1,3) at time t=1. | CO2 | 12 |
|  | b. | Deduce the expression for the Bernoulli’s equation stating clearly the assumptions involved. | CO1 | 8 |
| 7. | a. | A horizontal pipe of diameter 500 mm is suddenly contracted to a diameter of 250 mm. The pressure intensities in the large and smaller pipe is given as 13.734 N/cm2 and 11.772 N/cm2 respectively. Find the loss of head due to contraction if Cc=0.62. Also determine the rate of flow of water. | CO3 | 13 |
|  | b. | Analyse the different flow regimes that occur in a flow through a pipe by illustrating the Reynolds experiment with neat sketches. | CO3 | 7 |
| (OR) |  |  |  |  |
| 8. | a. | A main pipe divides into two parallel pipes which again forms one single pipe. The length and diameter of the first parallel pipe are 2000 m and 1.0 m respectively, while the length and diameter for the second parallel pipe are 2000 m and 0.8 m. Find the rate of flow in each parallel pipe, if the total flow in the main pipe is 3.0 m3/s. The coefficient of friction for each parallel pipe is same and equal to 0.005. | CO3 | 14 |
|  | b. | A pipe of diameter 300 mm and length 3500 m is used for the transmission of power by water. The total head at the inlet of the pipe is 500 m. Find the maximum power available at the outlet of the pipe, if the value of f=0.006. | CO3 | 6 |
|  |  | Compulsory: |  |  |
| 9. | a. | Derive the expression for discharge through the venturimeter? | CO2 | 14 |
|  | b. | A pitot tube is inserted in a pipe of 300 mm diameter. The static pressure in pipe is 100 mm of mercury(vacuum). The stagnation pressure at the centre of the pipe recorded by the pitot tube is 0.981 N/cm2. Calculate the rate of flow of water through the pipe if the mean velocity of the flow is 0.85 times the central velocity. Take coefficient of velocity as 0.98. | CO2 | 6 |