Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : 14CE2003 |  | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : MECHANICS OF FLUIDS |  | # Max. marks : | # 100 |

# (Biotech only)

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | An oil of viscosity of 0.5 N s/m2 is used to lubricate between a shaft and sleeve. The diameter of the shaft is 0.5 m and it rotates at 600 rpm. Measure the power loss for a sleeve length of 2 m. The thickness of oil film is 1 cm. | CO1 | 15 |
|  | b. | Estimate the diameter of a soap bubble when the inside absolute pressure is 86 cm Hg. Given atmospheric pressure is 76 cm Hg, and surface tension is 0.07 N/m. | CO1 | 5 |
| (OR) |  |  |  |  |
| 2. |  | An inverted U-tube manometer containing mercury is connected to two pipe carrying oil of sp. gr. 0.9, as shown in the figure. The differential manometer reading is 25 cm and the vertical distance between center of the pipes is 15 cm. Calculate pressure difference between the two pipes. | CO1 | 20 |
| 3. | a. | A 30 cm diameter pipe, conveying oil (sp. gr. 0.8), branches into two pipes of 30 cm and 15 cm respectively. If the average velocity in both the branches are 2 m/s, calculate average velocity in the main pipe. | CO2 | 10 |
|  | b. | Explain Bernoulli’s theorem (with mathematical expression) for steady flow of an incompressible fluid. | CO2 | 10 |
| (OR) |  |  |  |  |
| 4. |  | A horizontal venture meter with inlet and throat diameter of 30 cm and 20 cm is connected to a pipe carrying water. The pressure at inlet is 20 N/cm2and vacuum pressure at the throat is 10 cm mercury. Calculate the discharge rate, if Cd = 0.9. | CO2 | 20 |
| 5. |  | The rate of flow of water through a horizontal pipe is 0.5 m3/s. The diameter of the pipe is suddenly enlarged from 20 cm to 40 cm. If the pressure intensity in smaller pipe is 10 N/cm2, Estimate (i) pressure in the large pipe, (ii) power loss due to enlargement. | CO2 | 20 |
| (OR) |  |  |  |  |
| 6. |  | A horizontal pipe of 50 cm diameter is suddenly contracted to 25 cm diameter. U-tube mercury manometer connected to large and small pipe shows a reading of 30 cm. Calculate the flow rate of water, given Cc = 0.60. | CO2 | 20 |
| 7. |  | Water is flowing through a 50 m long vertical pipe of 20 cm diameter. The pressure at the bottom and top end of the pipe are 80 N/cm2 and 40 N/cm2, respectively. Estimate the flow direction and rate through the pipe, given f= 0.01 in formula | CO3 | 20 |
| (OR) |  |  |  |  |
| 8. |  | Determine the rate of flow of water through a pipe of diameter 20 cm and length 50 m when one end is connected to a reservoir and other end open to atmosphere. The pipe is horizontal and the water level in tank is 4 m above the center of pipe. Consider all minor losses and assume Darcy’s friction factor f = 0.009. | CO3 | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | Explain the difference between orifice and mouthpiece with appropriate diagram. | CO3 | 10 |
|  | b. | The head of water over a circular orifice of diameter 4 cm is 10 m. Calculate the actual velocity and discharge of the jet at vena-contracta. Assume, Cd and Cv equals to 0.6 and 0.9 respectively. | CO3 | 10 |