Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14CE2006 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # STRENGTH OF MATERIALS | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | A cantilever of length 3m carries a uniformly distributed load of 60 kN/m over the entire length. If E = 2.2x108kN/m2 and I = 109mm4 find the slope and deflection at the free end using conjugate beam method. | CO2 | 16 |
|  | b. | Define the terms slope and deflection. | CO2 | 4 |
| (OR) |  |  |  |  |
| 2. | a. | A beam of length 20m is simply supported at its ends and carries two point loads of 20kN and 30 kN at a distance of 7m and 12m from left end respectively. Calculate (i) deflection under each load (ii) maximum deflection. Take E = 2x106N/mm2 and I = 1.2x109 mm4. | CO2 | 16 |
|  | b. | State conjugate beam theorem I and II. | CO2 | 4 |
| 3. |  | A continuous beam ABC of length 10m rests on three supports A, B, and C at the same level in which span AB = 6m and span BC = 4m. In span A, there is a point load of 8kN at a distance of 2m from the end A, whereas in the span BC, there is a uniformly distributed load of 6kN/m run over the whole length. Determine the support moments and support reactions. Draw S.F.and B.M. diagrams. | CO1 | 20 |
| (OR) |  |  |  |  |
| 4. | a. | A beam of span 6m is fixed at its both ends. It carries 2 concentrated loads of 10 kN and 15kN each at a distance of 2m from both the ends. Find the fixing moments and draw the bending moment diagram. | CO1 | 16 |
|  | b | List the advantages and disadvantages of a fixed beam over a simply supported beam? | CO1 | 4 |
| 5. | a. | The external and internal diameter of a hollow cast iron are 5cm and 3cm respectively. If the length of this column is 4m and both of its end are fixed, determine the crippling load using Rankine’s formula. Take the value of fc= 550 N/mm2 and a = 11600 in Rankine’s formula. | CO2 | 15 |
|  | b | List the assumptions of Euler’s theory for long column analysis. | CO2 | 5 |
| (OR) |  |  |  |  |
| 6. |  | A simply supported beam of length 6m is subjected to a uniformly distributed load of 15 kN/m over the whole span and deflects 10 mm at the center. Estimate the crippling load when this beam is used as a column the following conditions: - one end is fixed and other end hinged. - both the ends pin jointed. - Both the ends fixed. | CO2 | 20 |
| 7. | a. | A pipe of 200mm internal diameter and 50mm thickness carries fluid at a pressure of 10MPa. Determine the maximum and minimum circumference stresses across the section. Also sketch the radial stress (pressure) distribution and circumferential stress distribution across the section. | CO1 | 8 |
|  | b. | A thin cylindrical shell of 120cm diameter, 1.5cm thick and 6m long is subjected to internal fluid pressure of 2.5N/mm2. If the value of E = 2 x 105 N/mm2 and poisson’s ratio = 0.3, calculate the change in diameter, change in length and change in volume. | CO1 | 12 |
| (OR) |  |  |  |  |
| 8. | a. | A cantilever of I section 2.4m long is subjected to a load of 20kN at the free end. Determine the resulting bending stress at corners of the top flange on the fixed end of the cantilever. The load is inclined at 20o to the vertical and passing through the centroid of the section. Thickness of web=2mm. thickness of flange = 2.5 mm. 50 mm 30 mm A C D B 2.5 mm 2.5 mm 20 P = 20k N X XU X XU Y Y YU YU | CO2 | 16 |
|  | b. | Locate the shear centre of a channel section of uniform thickness, b = 120mm, h = 200mm, t = 4mm. | CO2 | 4 |
|  |  | Compulsory: |  |  |
| 9. |  | Give the significance of “theories of failure”? Name any five important theories of failure. Explain in detail the following theories of Failure. - Maximum Principal stress theory ( due to Rankine). - Maximum shear stress theory ( Guest - Tresca ). - Total strain energy per unit volume ( Haigh ) Theory. | CO3 | 20 |