Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14CE2006 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # STRENGTH OF MATERIALS | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | A steel girder of uniform section 14m long is simply supported at its ends. It carries concentrated loads of 90kN and 60kN at two points 3m and 4.5m from the ends. Calculate: i. the deflections of the girder at the points under the loads ii. the maximum deflection. Take I = 64x108mm4 and E = 210kN/mm2. | CO2 | 14 |
|  | b. | State the application of conjugate beam method. Differentiate actual beam and conjugate beam. | CO2 | 6 |
| (OR) |  |  |  |  |
| 2. | a. | Calculate the value of maximum deflection in the beam shown below. Evaluate the slope at supports. EI is constant. Use Macaulay’s method. Take EI= 9600kN-m2 | CO2 | 14 |
|  | b. | State moment area theorem I and II. | CO2 | 6 |
| 3. |  | Draw the shear force and bending moment diagrams of the given continuous beam using Clapeyrons theorm. | CO1 | 20 |
| (OR) |  |  |  |  |
| 4. |  | A continuous beam ABCD, simply supported at A,B,C and D is loaded as shown in fig. Sketch the B.M. and S.F. diagrams. | CO1 | 20 |
| 5. |  | A hollow cast iron column whose outside diameter is 200mm has a thickness of 20mm. It is 4.5m long and is fixed at both ends. Calculate the safe load by Rankine’s formula using a factor of safety of 4. Evaluate the ratio of Euler’s and Rankine’s Buckling loads. Taking σc = 550N/mm2 , α = 1/1600 and E = 9.4x104N/mm2. | CO2 | 20 |
| (OR) |  |  |  |  |
| 6. |  | A simply supported beam of length 4m is subjected to a uniformly distributed load of 30 kN/m over the whole span and deflects 15 mm at the center. Estimate the crippling load when this beam is used as a column with the following conditions: i)one end is fixed and other end hinged. ii)both the ends pin jointed. iii) Both the ends fixed. | CO2 | 20 |
| 7. | a. | A thick spherical shell of 200mm internal diameter is subjected to a internal fluid pressure of 7N/mm2.If the permissible tensile stresses in the shell material is 8N/mm2, find the thickness of the shell. | CO1 | 8 |
|  | b. | Estimate the change in i. Diameter ii. Length and iii. Volume of a thin cylindrical shell of 100cm diameter, 1cm thickness and 5m length when subjected to an internal fluid pressure of 4N/mm2. Take the value of E= 2x105 N/mm2 and Poisson’s ratio = 0.3. | CO1 | 12 |
| (OR) |  |  |  |  |
| 8. | a. | A beam of T section (flange: 100mm x 20mm; web: 150mm x 10mm) is 2.5m long and is simply supported at the ends. It carries a concentrated load of 3.2kN at its centre but inclined at an angle of 20o to the vertical and passing through the centroid of the section. If E = 200GPa, determine the maximum tensile stress, maximum compressive stress. | CO2 | 12 |
|  | b. | Define shear centre. Give its significance. Locate the shear centre of a channel section of of uniform thickness, b = 100mm, h = 150mm, t = 3mm. | CO2 | 8 |
|  |  | Compulsory: |  |  |
| 9. |  | Explain in detail the following theories of Failure. a. Maximum Principal stress theory. ( due to Rankine ). b. Maximum shear stress theory. ( Guest - Tresca ). c. Total strain energy per unit volume. ( Haigh ) Theory. | CO3 | 7+7+6 |