Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14CE2007 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # SOIL MECHANICS | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Describe the laboratory method for the determination of relative density. | CO1 | 5 |
|  | b. | A sample of wet soil has a volume of 0.0192 m3 and a mass of 32 kg. When the sample is dried out in an oven, its mass reduced to 28.5 kg. Determine (i) the bulk density (ii) water content (iii) dry density (iv) saturated density (v) void ratio (vi) porosity (vii) degree of saturation. Assume the specific gravity of solids as 2.65. | CO1 | 15 |
| (OR) |  |  |  |  |
| 2. | a. | Derive the relation between ‘water content (w)’, ‘Specific Gravity (Gs)’, ‘void ratio(e)’, and ‘Degree of Saturation(Sr)’. | CO1 | 5 |
|  | b. | Draw the particle size determination curve and determine the Cu and Cc. Sieve; 2; 1; 0.6; 0.425; 0.212; 0.15; 0.075; Pan. Mass of soil retained (g); 0; 2.02; 3.51; 7.53; 8.15; 2.81; 0.9; 0.08. | CO1 | 15 |
| 3. | a. | Write a note on IS classification of soil. | CO1 | 5 |
|  | b. | Write the experimental procedure to determine the Liquid and Plastic limits of clayey soil. | CO1 | 15 |
| (OR) |  |  |  |  |
| 4. | a. | Explain quick sand. | CO1 | 5 |
|  | b. | List out the factors affecting the Permeability? | CO1 | 5 |
|  | c. | A soil formation has three layers with permeabilities 0.02cm/sec for first 3m, 3 x 10-4m/sec for next 1.0m, 0.06 cm/sec for next 1.5m and 5 x 10-7 m/sec for 3m thickness. Estimate the average coefficient of permeability. Assume the flow is taking place perpendicular and parallel to the bedding plane. | CO1 | 10 |
| 5. | a. | What are the factors affecting the compaction. | CO2 | 5 |
|  | b. | The following results were obtained from a standard compaction test performed on a sample of soil. WC (%); 7.7; 11.5; 14.6; 17.5; 19.7; 21.2. Mass (kg); 1.7; 1.89; 2.03; 1.99; 1.96; 1.92. The Volume of the mould used was 950c.c. and the specific gravity of soil grains was 2.65, make a necessary calculations and plot the water content-dry density curve and obtain the optimum water content and the maximum dry density. | CO2 | 15 |
| (OR) |  |  |  |  |
| 6. | a. | Differentiate between the normally consolidated, over consolidated and under consolidated soils. | CO2 | 5 |
|  | b. | A stratum of clay is 4m thick and has an initial overburden pressure of 45 kN/m2 at its middle. Determine the final settlement due to an increase in pressure of 35 kN/m2 at the middle of the clay layer. The clay is over-consolidated, with a preconsolidation pressure of 60 kN/m2. The values of the coefficients of recompression and compression index are 0.05 and 0.25 respectively. Take initial void ratio as 1.40. | CO2 | 15 |
| 7. | a. | Differentiate between primary consolidation and secondary consolidation. | CO2 | 5 |
|  | b. | The water table in a deposit of sand 8 m thick is at a depth of 3m below the surface. The bulk density of sand is 19.62 kN/m3. Calculate the effective pressure of 1m, 5m and 8m below the surface. Hence plot the variation of total pressure, neutral pressure and effective pressure over the depth of 8 m. | CO2 | 15 |
| (OR) |  |  |  |  |
| 8. | a. | Write Notes on Iso-Bars with neat sketch. | CO3 | 5 |
|  | b. | List the different types of slope failures? Give the different types of safety used in the stability of slopes? | CO3 | 15 |
|  |  | Compulsory: |  |  |
| 9. |  | Un-drained triaxial tests are carried out on four identical specimens of silt clay, and thefollowing results are obtained: Sample No.; Cell pressure ( kN/m2); Deviator stress at failure ( kN/m2); Pore pressure ( kN/m2). 1; 50; 350; 5. 2; 100; 440; 10. 3; 150; 530; 12. 4; 200; 610; 18. Determine the value of the effective angles of shearing resistance and the cohesion intercept by plotting conventional failure envelope. | CO3 | 20 |