Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14CE2007 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # SOIL MECHANICS | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Derive the relationship between Porosity and Void ratio. | CO1 | 5 |
|  | b. | Differentiate between dry density, Bulk density, Saturated density and Submerged density. | CO1 | 5 |
|  | c. | A moist soil sample weighs 3.52 N. After drying in an oven, its weight is reduced to 2.9 N. The specific gravity of solids and the mass specific gravity are 2.65and 1.85 respectively. Determine the water content, void ratio, porosity and the degree of saturation. Take γw = 10 kN/m3. | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Briefly explain about the two phase and three phase block diagram? | CO1 | 5 |
|  | b. | An air dry soil sample weighing 25kg was sieved in a laboratory. The results are given below. IS sieve (mm); 2.0; 1.0; 0.6; 0.425; 0.212; 0.150; 0.075; Pan. Mass retained; 0; 2.02; 3.51; 7.53; 8.15; 2.81; 0.90; 0.08. Draw the grain size distribution curve and determine the percentage of gravel, sand and silt clay. | CO1 | 15 |
| 3. | a. | Describe the laboratory method for the determination of specific gravity. | CO1 | 5 |
|  | b. | Describe the method for determination of liquid and plastic limit of soil? | CO1 | 15 |
| (OR) |  |  |  |  |
| 4. |  | Elaborate the different methods for the determination of the coefficient of permeability in the laboratory? | CO1 | 20 |
| 5. | a. | Relate the effect of seepage pressure with hydraulic gradient. | CO2 | 5 |
|  | b. | Describe the standard proctor test and the modified proctor test. How would you decide the type of test to be conducted in the laboratory? | CO2 | 15 |
| (OR) |  |  |  |  |
| 6. | a. | Describe the consolidometer test. Show how the results of this test are used to predict the rate of settlement and the magnitude of settlement. | CO2 | 10 |
|  | b. | A 3m thick clay layer beneath a building is overlain by a permeable stratum and is underlain by an impervious rock. The coefficient of consolidation of the clay was found to be 0.025cm2/min. The final expected settlement for the layer is 8cm. - How much time will it take for 80% of the total settlement to take place? - Determine the time required for a settlement of 2.5cm to occur. - Compute the settlement that would occur in one year. | CO2 | 10 |
| 7. | a. | What is effective stress equation? What is it impact on shear strength? | CO3 | 5 |
|  | b. | Calculate the total, neutral and effective stress at the bottom of a soil deposit having unsaturated layer of sand of 16 kN/m3 to a depth of 2.5 m from the ground level, saturated layer of silt of 19 kN/m3 to a depth of 3m below the sand and bottom saturated clay of 18 kN/m3 to a depth of 4.5 m. | CO3 | 15 |
| (OR) |  |  |  |  |
| 8. |  | Discuss the basis of the construction of Newmark’s influence chart. How is it used? | CO3 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | The following results were obtained from a series of consolidated undrained test on soil, in which the pore water pressure was not determined. Determine the cohesion intercept and the angle of shearing resistance. Sample No.; Confining Pressure (kN/m2); Deviator Stress at failure ( kN/m2). 1; 150; 500. 2; 300; 650. 3; 450; 800. | CO3 | 20 |