Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14CE2008 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # WATER AND WASTE WATER ENGINEERING | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Elaborate the different parameters of total quantity of water required for a town. | CO1 | 10 |
|  | b. | The population of growing town are as follows. Estimate the population by the year 2030 with arithmetic and geometric increase methods. Year; 1980; 1990; 2000; 2010. population; 40000; 50000; 80000; 140000. | CO2 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Compile procedure for the determination of alkanity and hardness present in the water in laboratory. | CO2 | 15 |
|  | b. | List out the standards for different 10 water quality parameters according to BIS. | CO1 | 5 |
| 3. |  | Explain the importance of following water quality parameters - Turbidity - Solids in water - Chlorides - Acidity - Fluoride content - Dissolved oxygen - Hardness - Color and odour | CO2 | 20 |
| (OR) |  |  |  |  |
| 4. | a. | Different types of sedimentation tanks with neat sketches. | CO1 | 7 |
|  | b. | A sedimentation tank designed to capture 0.01 mm diameter particles with specific gravity 2.65. If sedimentation tank is designed for an surface over flow rate 20 m3/day/m2. Find the efficiency of particle removal. Take kinematic viscosity of water 0.01 cm2/sec. | CO2 | 7 |
|  | c. | Formulate different types of settling velocity formulae according to the particle size with each parameter. | CO2 | 6 |
| 5. | a. | Outline the treatment process of river water for the drinking purpose with neat sketch and their importance with respect to removal impurity. | CO1 | 12 |
|  | b. | Differentiate different types of disinfection methods and their importance. | CO2 | 8 |
| (OR) |  |  |  |  |
| 6. | a. | Paraphrase about layout of distribution networks. | CO2 | 10 |
|  | b. | What do you understand by continuous and intermittent supply systems of water? Compare both with respect to their merits and demerits. | CO3 | 10 |
| 7. | a. | Estimate the dry weather flow to calculate the design discharge. | CO3 | 5 |
|  | b. | Elaborate about ultimate disposal of sewage. | CO3 | 5 |
|  | c. | Write short notes on grit chamber and screens. | CO1 | 5 |
|  | d. | What is coagulation? List the common coagulation used in water treatment process. | CO1 | 5 |
| (OR) |  |  |  |  |
| 8. | a. | Clearly bring out the comparison between activated sludge process and trickling filter. | CO2 | 10 |
|  | b. | Differentiate between ‘Sewage’ and ‘Sullage ’? Discuss the rational formula for calculating the peak drainage discharge from a given catchment and reaching a particular storm water drain up to a particular point. | CO2 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | What are the differences between slow and rapid sand filter? Distinguish between slow sand gravity and rapid sand gravity filters with reference to rate of filtration, quantity of sand and size of filter, main treatment process responsible, pretreatment required, quantity of water required for cleaning, operation problems involved, maintenance required, period of cleaning and method of cleaning, loss of head, efficiency of removal of bacteria, efficiency of removal of turbidity and suitability in water supply schemes. | CO2 | 15 |
|  | b. | Explain the significance of fluctuations in water demand. | CO3 | 5 |