Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14CE2009 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # REINFORCED CONCRETE STRUCTURES | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Draw the stress strain curve for steel & concrete and mark the salient points. | CO1 | 4 |
|  | b. | Discuss the various failure theories of beams with neat sketches | CO1 | 16 |
| (OR) |  |  |  |  |
| 2. |  | Design the slab located in the corner of the building of size 5 x 3.5 m supported on 230mm walls. Assume live load as 4kN/m2 and floor finish of 1kN/m2. Use M30 concrete and Fe415 steel. (Edge condition: Two adjacent edges discontinuous). | CO2 | 20 |
| 3. |  | A simply supported beam of size 300x500mm effective depth has to resist a factored moment of 300 kNm and a factored shear of 150kN. Use M20 concrete and Fe415 steel. Design the reinforcement and the stirrups in the beam. | CO2 | 20 |
| (OR) |  |  |  |  |
| 4. |  | Design a beam of span 4.5m supported on walls 230mm thick. The load on the beam is 20kN/m. Use M25 concrete and Fe415 steel. | CO2 | 20 |
| 5 |  | Find the moment of resistance of T-beam and suggest the economical section, having the following data. bf = 700mm, d= 600mm, bw= 240mm, Df= 90mm. Use M20 concrete and Fe415 steel, when a. Ast= 2454 mm2 b. Ast = 1200mm2 | CO3 | 20 |
| (OR) |  |  |  |  |
| 6. | a. | Differentiate singly and doubly reinforced beam section? | CO2 | 4 |
|  | b. | Mention the minimum percentage of reinforcement to be used in columns? Also write the minimum number of rods to be provided in square and circular columns? | CO2 | 2 |
|  | c. | Design the column of size 230x600mm with effective length of 4m capable of safely resisting a Factored load of Pu=1500kN and Factored moment of 200kNm. Use M20 concrete and Fe415 steel. | CO2 | 14 |
| 7. | a. | Write the effective length of column effectively held in position at both ends and restrained against rotation at one end? | CO2 | 1 |
|  | b. | Mention the condition for providing side face reinforcement in beam? | CO2 | 1 |
|  | c. | Differentiate T-beam section and L-beam section | CO2 | 2 |
|  | d. | List the various limit states to be considered in the design of RC elements | CO3 | 2 |
|  | e. | Design a square column of size 300x300mm with effective length of 3.5m to resist a factored load of Pu=600kN and an eccentricity of 75mm. Use M20 concrete and Fe415 steel. | CO2 | 14 |
| (OR) |  |  |  |  |
| 8. |  | An RC column of size 450 x 450mm carries an axial load of 1200kN. Design the footing for the column using M20 concrete and Fe415 steel. The safe bearing capacity of soil may be taken as 300kN/m2. | CO2 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | Design the stem of a cantilever retaining wall to retain earth embankment 3.5 m high above the ground level. The unit weight of earth is 18kN/m3 and its angle of repose is 30o. The embankment is horizontal at its top. The safe bearing capacity of soil may be taken as 150kN/m2 and the coefficient of friction between soil and concrete as 0.5. Use M20 concrete and Fe415 steel. | CO2 | 20 |