Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14CE2010 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # STRUCTURAL ANALYSIS | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Analyse the frame shown in Fig. by virtual work method and determine the horizontal displacement at support D. Take E= 200x106kN/m2. I = 300x10-6 m4. | CO1,CO3 | 12 |
|  | b. | Calculate the deflection at the midspan of a simply supported beam of span L when it carries a UDL wkN/m throughout the span using Castigliano’s first theorem. | CO1 | 8 |
| (OR) |  |  |  |  |
| 2. |  | Find the vertical deflection at joint B of the pin-jointed frame shown in figure. AE is constant for all members. | CO1,CO3 | 20 |
| 3. |  | A continuous beam is supported and loaded as shown in figure. Analyse the beam for support moments and reactions. Take E = 200 x 106kN/m2 and I=100x106 mm4 constant throughout. Use slope-deflection method. | CO3 | 20 |
| (OR) |  |  |  |  |
| 4. |  | A continuous beam ABCD, 11 m long, is fixed at ends A and D. The first span of length 4 m is loaded with a point load of 8 kN acting at 1.5 m from left support, the second span of length 4 m is loaded with a UDL of intensity 3 kN/m and the third is loaded with a point load of 6 kN acting at 2 m from right support. Spans AB, BC and CD have moments of Inertia of I, 1.5I and I respectively and are of the same material. Using the slope deflection method, calculate the moments at the supports, if the support B sinks by 5 mm downwards. Take I = 93×106 mm4 and E= 2.1×105 N/mm2. | CO3 | 20 |
| 5. |  | A continuous beam ABCD consists of 3 spans and is loaded as shown in the figure. Ends A and D are fixed. Determine the bending moments at the supports and plot the BMD and SFD using moment distribution method. | CO3 | 20 |
| (OR) |  |  |  |  |
| 6. |  | A continuous beam ABC is supported on an elastic column BD and is loaded as shown in figure. Treating joint B as rigid, analyse the frame and plot BMD and SFD using Moment distribution method | CO3 | 20 |
| 7. |  | Two point loads of 90kN and 120kN spaced 3.5m apart cross a girder of span 20m from left to right with the 90kN leading. Determine the maximum values of shear force and bending moment that can occur at any point of the girder using influence lines. Also plot the maximum positive and negative shear force and bending moment diagramsstating their absolute maximum values. | CO2 | 20 |
| (OR) |  |  |  |  |
| 8. |  | A train of wheel loads as shown crosses a girder of 25m span with 120kN load leading. Wheel load (kN); 180; 160; 160; 120. Spacing (m); 2m; 2m; 3m. Determine the maximum B.M at a section 8m from the left end and the absolute maximum B.M on the girder. | CO2 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | Analyse the frame shown in Figure and evaluate approximately the column end moments, beam end moments and reactions. | CO3 | 20 |