Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14CE2011 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # WATER RESOURCES ENGINEERING | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Elaborate orographic, frontal and convective precipitation with supporting sketches. | CO3 | 10 |
|  | b. | A catchment has 6 rain gauge stations. In a year, the annual rainfall recorded by rain gauges are as follows. Station; A; B; C; D; E; F. Rainfall (cm); 82.6; 102.9; 180.3; 110.3; 98.8; 136.7. For a 10% error in the estimation of the mean rainfall, calculate the optimum number of stations in the catchment | CO3 | 10 |
| (OR) |  |  |  |  |
| 2. |  | Sketch and explain the different parts of a hydrograph. State the different types and uses of hydrographs. Also, define unit hydrograph. | CO3 | 20 |
| 3. | a. | Write a critique on i. Pumping Tests ii. Recuperation Test Draw the necessary sketches. | CO1 | 10 |
|  | b. | Design an open well in coarse sand for a yield of 0.04cumecs when operated under a depression head of 3m. Assume C’/A = 1.0 m3/hr/m2/m. | CO2 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | Summarise various properties of an aquifer. | CO1 | 10 |
|  | b. | A 35cm diameter well is fully penetrated into a confined aquifer of width b=30m. When pumped at a steady rate of 35 liters/min, the drawdowns observed in wells at a radial distances of 15m and 45m are 1.6m and 0.8m respectively. Compute the radius of influence, permeability, transmissibility and drawdown in the well. | CO2 | 10 |
| 5. | a. | Find the delta of a crop when its duty is 1000 hectares/cumec on the field, the base period of crop being 180 days. Also find the duty of delta if the delta increases by 30% and base period is reduced by 25 days. | CO2 | 10 |
|  | b. | Report the advantages and disadvantages of open wells. | CO1 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Comprehend on types of “open wells in unconsolidated formations”. | CO1 | 10 |
|  | b. | Brief about 1) Artificial recharge 2) Conjunctive use of surface and ground water. | CO2 | 10 |
| 7. | a. | Define the “per capita demand” and summarise the methods for forecasting the “future population” ? | CO2 | 12 |
|  | b. | Cite and extend the limits, sources, effects and treatment for hardness in drinking water. | CO1 | 8 |
| (OR) |  |  |  |  |
| 8. | a. | Write down and explain the different investigations involved in reservoir planning. | CO1 | 8 |
|  | b. | In a town, it has been decided to provide 70 liters/day in 21st century. Estimate the domestic water supply requirement of this town in year 2050 by projecting the population of the town from following data. Year Population 1925 2,50,000 1950 4,80,000 1975 5,50,000 2000 6,38,000 2025 6,95,000 Use - Arithmetic increase method. - Geometric increase method. - Incremental increase method. | CO2 | 12 |
|  |  | Compulsory: |  |  |
| 9. | a. | Illustrate the different types of reservoirs. | CO2 | 6 |
|  | b. | Classify turbines and explain any one. | CO1 | 6 |
|  | c. | Comment on causes and mitigation measures of pollution of drinking water. | CO3 | 4 |
|  | d. | Cite the sources and effects of heavy metals in drinking water. | CO3 | 4 |