Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14CE2012 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # FOUNDATION ENGINEERING | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Discuss the stages involved in sub surface explorations? | CO1 | 5 |
|  | b. | Briefly explain about the various geophysical methods with neat sketches. | CO1 | 15 |
| (OR) |  |  |  |  |
| 2. | a. | Discuss in detail about the salient features of the bore log report. | CO1 | 5 |
|  | b. | Describe the open excavation methods of exploration. | CO1 | 5 |
|  | c. | Outline the following methods of drilling holes for subsurface investigations with neat sketches. - Auger Boring. - Wash Boring. | CO1 | 10 |
| 3. | a. | Illustrate the conditions where a pile foundation is more suitable than a shallow foundation? | CO1 | 8 |
|  | b. | Briefly explain about the cone penetration test with neat sketch? How these tests differ from standard penetration test? | CO1 | 12 |
| (OR) |  |  |  |  |
| 4. | a. | A SPT was conducted in a dense sand deposit at a depth of 22 m, and a value of 48 was observed for N. The density of the sand was 15 kN/m2. What is the value of N, corrected for overburden pressure? | CO2 | 5 |
|  | b. | Briefly explain about the various types of soil samplers for obtaining the disturbed and undisturbed samples. | CO2 | 15 |
| 5. |  | A 2m x 2m footing is located at a depth of 1.5m from ground surface in dense sand. The relative shear parameters, cohesion c = 0, angle of internal friction  = 36. Determine the ultimate bearing capacity of the soil if, - water table is well below the foundation level. - water table is at the surface. - water table is at the base of footing. Take the unit weight of soil as 18 kN/m3 and saturated unit weight of soil as 20kN/m3. | CO2 | 20 |
| (OR) |  |  |  |  |
| 6. | a. | Describe the Meyerhof’s bearing capacity theory. How does it differ from Terzhagi’s theory? | CO3 | 8 |
|  | b. | Analyze the Terzhagi’s equation for the strip footing . Also how the water table affects the Terzhagi’s bearing capacity equation at different levels | CO3 | 12 |
| 7. | a. | Describe plate load test. What are its limitations and used? | CO3 | 10 |
|  | b. | Determine the active earth pressure on a retaining wall of height 6m retaining a leveled backfill. The angle of internal friction of the soil is 28°. In the first 3m from ground level the unit weight of the soil is 16kN/m3 and for the next 3m,the unit weight of saturated soil is 20kN/m3. Draw the pressure distribution diagram. Calculate the resultant pressure and its position. | CO3 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | Compare between Rankine’s theory and Columb’s theory of earth pressure. | CO3 | 10 |
|  | b. | Design an isolated footing for a column of 450mm x 450mm size subjected to a vertical load of 2500kN, moment of 400kN-m and shear load of 360 kN, Take allowable soil pressure is 1400kN/m2 | CO4 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | Define negative skin friction. What is its effect on the pile? | CO4 | 5 |
|  | b. | Design a square pile group to carry 400 kN in clay with an unconfined compression strength of 60 kN/m2. The piles are 30 cm diameter and 6 m long. Adhesion factor may be taken as 0.6. | CO4 | 15 |