Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14CE2013 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # DESIGN OF STEEL STRUCTURES | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. |  | Sub Div. | Questions | Course Outcome |  | Marks |
| --- | --- | --- | --- | --- | --- | --- |
| 1. |  | a. | Elaborate on the classifications of structural steel sections. | CO1 |  | 5 |
|  |  | b. | Estimate the strength of a double cover butt joint for connecting two plates of 10mm thickness connected by cover plates of thickness 8mm. Use M20 bolts of grade 4.6 and Fe 410 plates are to be used. | CO2 |  | 15 |
| (OR) |  |  |  |  |  |  |
| 2. |  | a. | List the advantages of welded connections. | CO1 |  | 5 |
|  |  | b. | A tie member of a roof truss consists of 2 ISA 100 x 75 x 8mm. The angles are connected to either side of a 10mm gusset plates and the member is subjected to a working pull of 300kN. Design the welded connection. Assume the connections are made in the workshop. | CO2 |  | 15 |
| 3. |  | a. | Bring out the design and construction requirements to be satisfied in the design of steel structures. | CO3 |  | 5 |
|  |  | b. | Calculate the maximum tension allowed on a single angle ISA 75 × 75 × 6 if it is connected on one side of gusset plate of 8mm thick with bolts of 20mm diameter. Bolt strength is 43kN per bolt. End distance is 35mm and pitch for bolts is 50 mm. | CO2 |  | 15 |
| (OR) |  |  |  |  |  |  |
| 4. |  | a | Can you state when a tension in a tension member may become nonfunctional? | CO1 |  | 4 |
|  |  | b | Point out how the strength of members subjected to axial compression can be defined? | CO1 |  | 4 |
|  |  | c. | A column of an industrial building bears an working load of 800kN. The column of height 6m can be assumed to be pinned at supports. Design a column using a suitable ISMB section. Use fy = 250 MPa. | CO2 |  | 12 |
| 5. |  | a. | Bring out the reason for preferring plastic or compact section for flexural members in limit state design method. | CO1 |  | 5 |
|  |  | b. | A simply supported steel beam of 4.0m effective span is laterally supported throughout. It carries a total uniformly distributed load of 40kN/m (inclusive of self weight). Design an appropriate section using steel of grade Fe 410. | CO2 |  | 15 |
| (OR) |  |  |  |  |  |  |
| 6. |  | a. | Can you state how you determine the nominal bearing pressure between the base plate and support below? |  | CO1 | 5 |
|  |  | b. | Suggest a suitable column base for an ISMB 300 column section to bear a load of 750 kN (factored) reaction. Design the base plate to be supported on a pedestal of M20 grade of concrete. |  | CO2 | 15 |
| 7. |  | a. | Design a welded plate girder for a simply supported bridge deck beam with clear span of 20m,subjected to the following, Dead load including self weight =20kN/m, Imposed load = 10kN/m Assume that the top compression flange of the plate girder is restrained laterally. Design as an unstiffened plate girder. |  | CO2 | 20 |
| (OR) |  |  |  |  |  |  |
| 8. | a. |  | Sketch the different types of roofs being using in Industrial building for different spans. | CO3 |  | 5 |
|  | b. |  | Determine the dead, live and wind load of a fink type roof truss for an industrial building for the following data. Width of the building = 12 m Length of the building = 40 m c/c spacing of trusses = 5 m Rise of truss = ¼ of span Self weight of purlins = 150N/m Height of column = 8 m Roofing and side covering = Asbestos cement sheets (Dead weight =125N/m2) The building is located in industrial area Naini, Allahabad. Both the ends of the truss are hinged | CO2 |  | 15 |
|  |  |  | Compulsory: |  |  |  |
| 9. |  |  | Design a laced column 10m long to carry a factored axial load of 1500kN.The column is restrained in position but not in direction at both ends. Provide single lacing system with bolted connection. Design the column with two channels placed toe to toe. Assume grade of steel Fe 410 and bolts of grade 4.6. | CO2 |  | 20 |