Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14CE2014 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # TRANSPORTATION ENGINEERING | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

(Tables & Charts for Design of Flexible Pavement may permitted)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Paraphrase the requirements of highway alignment. | CO1 | 8 |
|  | b. | Define the term kinks in a rails. | CO2 | 7 |
|  | c. | Lists the factors which influence the location of an airport. | CO6 | 5 |
| (OR) |  |  |  |  |
| 2. | a. | Discuss special care to be taken in highway alignment. | CO1 | 6 |
|  | b. | Recognize Elastic Fastening and its types. | CO2 | 5 |
|  | c. | Draw the typical layout of airport components. | CO6 | 9 |
| 3. | a. | Write short note on - Jayakar Committee. - Indian Road Congress. - Central Road Fund. | CO1 | 3+3+2 |
|  | b. | Illustrate with neat sketches, functions, types and principles of marshalling yards. | CO2 | 6 |
|  | c. | Summarize wind rose diagram and its methods. | CO6 | 6 |
| (OR) |  |  |  |  |
| 4. | a. | The speeds of overtaking and overtaken vehicles are 80 and 60 kmph respectively. If the acceleration of overtaking vehicles is 2.5 kpmh per second. Calculate the safe overtaking sight distance. | CO3 | 10 |
|  | b. | Draw a typical cross section of a permanent way and indicate its components. Explain various requirements and functions of each component. | CO2 | 10 |
| 5. | a. | Enumerate the steps for practical design of super elevation. | CO5 | 7 |
|  | b. | Point out the different geometric elements of railway track. | CO2 | 7 |
|  | c. | Construct the approaches of runway geometric design. | CO6 | 6 |
| (OR) |  |  |  |  |
| 6. | a. | Calculate the length of transition curve for a design speed of 80kmph at horizontal curve of radius 300 m in a rural area. Assume suitable data. | CO3 | 10 |
|  | b. | State track circuiting and its working principles. | CO2 | 10 |
| 7. | a. | A vertical summit curve is formed when an ascending gradient of 1 in 20 meets another ascending gradient of 1 in 100. Find the length of the summit curve to provide the required stopping sight distance for a design speed of 80kmph. | CO3 | 7 |
|  | b. | Draw a sketch of flexible pavement cross section and show the component parts. | CO4 | 6 |
|  | c. | Mention the various marking used on runway. | CO6 | 7 |
| (OR) |  |  |  |  |
| 8. | a. | Distinguish between stations and yards. | CO2 | 7 |
|  | b. | Design a flexible pavement for a new by pass road for a Two-lane single carriageway of initial traffic in a year of completion of construction is 800 CV/day (sum of both directions). Traffic growth rate per annum is 7.5 percent. Design life of 10 years, with the value of vehicle damage factor is 2.5 (standard axles per commercial vehicle) and design CBR value of sub-grade soil is 6 %. | CO4 | 13 |
|  |  | Compulsory: |  |  |
| 9. | a. | Construct layout of horbour and illustrate with neat sketch | CO6 | 10 |
|  | b. | Identify the facilities should provide at major port | CO6 | 10 |