Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14CE2014 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # TRANSPORTATION ENGINEERING | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

(Tables & Charts for Design of Flexible Pavement may permitted)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Compare the characteristics features of different modes of transportation. | CO1 | 9 |
|  | b. | Label the types of switches. | CO2 | 5 |
|  | c. | Describe the regional planning and advantages in airport planning. | CO6 | 6 |
| (OR) |  |  |  |  |
| 2. | a. | Explain obligatory points. | CO1 | 8 |
|  | b. | Discuss Major railway track maintenance operations. | CO2 | 8 |
|  | c. | Mention the approaches of development of new airport. | CO6 | 4 |
| 3. | a. | Discuss special care to be taken while aligning hill roads. | CO5 | 6 |
|  | b. | Explain with neat sketches the layout of various stations indicating various passenger facilities. | CO6 | 14 |
| (OR) |  |  |  |  |
| 4. | a. | Indicate total reaction time of the driver and the factors on which it depends. Explain “PIEV Theory”. | CO1 | 8 |
|  | b. | Lists the various constituents of a turnout and state its functions. | CO2 | 12 |
| 5. | a. | Calculate stopping sight distance for the design speed of 100kmph. Take total reaction time = 2.5 sec. Coefficient of friction = 0.35. | CO3 | 6 |
|  | b. | Explain with neat sketches, the concept of coning and discuss its merits and demerits. | CO2 | 7 |
|  | c. | Generalize the working principles of instrumental landing system. | CO6 | 7 |
| (OR) |  |  |  |  |
| 6. | a. | Calculate the extra widening of the pavement required on horizontal curve of radius 700m on a two lane highway, the design speed being 80 kmph. Assume wheel base l = 6m. | CO3 | 7 |
|  | b. | Illustrate with neat sketch the location of various signals at a railway station. | CO2 | 7 |
|  | c. | Compile different types of Airport Marking. | CO6 | 6 |
| 7. | a. | An ascending gradient of 1 in 50 meets a descending gradient of 1 in 80. Determine the length of summit curve to provide (a) ISD (b) OSD for design speed of 80kmph. Assume all other data. | CO3 | 8 |
|  | b. | Explain Flexible and Rigid pavements and bring out the points of difference. | CO4 | 7 |
|  | c. | Point out the various marking used on taxiway. | CO6 | 5 |
| (OR) |  |  |  |  |
| 8. | a. | Classify the railway station based on their characters. | CO2 | 10 |
|  | b. | Design a flexible pavement for a new bypass road for a Two-lane single carriageway of initial traffic in a year of completion of construction is 600 CV/day(sum of both directions). Traffic growth rate per annum is 7 percent. Design life of 15 years, with the value of vehicle damage factor is 2.5 (standard axles per commercial vehicle) and design CBR value of sub-grade soil is 4 %. | CO4 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | Express the requirements of Harbour | CO6 | 10 |
|  | b. | State and explain with neat sketch about the classification of docks | CO6 | 10 |