Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14CE2029 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ADVANCED REINFORCED CONCRETE STRUCTURES | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

IS Codes permitted for the exam : IS456:2000, IRC:21, IRC:6

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Enlist the advantages and disadvantages of flat slab over normal roof slab. | CO1 | 4 |
|  | b. | Design a roof system without beams for the interior panel of size 4m x 4m, for live load of 3 kN/m2 . The floor supports column of size 350mm x 350mm. Sketch the reinforcement diagram for the slab system. | CO2 CO3 | 14 2 |
| (OR) |  |  |  |  |
| 2. | a. | In the design of a flat slab punching shear is more, enumerate the measures to be taken to prevent punching shear. | CO1 | 4 |
|  | b. | Design the exterior panel of a flat slab of size 5m x5m with suitable drop to support a live load of 4kN/m2. The floor system is supported by columns of size 400 x 400 mm . Floor height is 3.2 m. Use M20 concrete and Fe 415 steel. Sketch the reinforcement diagram for the slab system. | CO2 CO3 | 14 2 |
| 3. | a. | Sketch the loading pattern of class B loading. | CO1 | 2 |
|  | b. | Design a simply supported RC slab required for the deck of a road bridge having the following data: Clear width of roadway = 7.5 m Foot paths = 1m on either side Clear span = 6 m Width of bearing = 600mm Thickness of wearing coat = 80mm Type of loading : IRC class AA Materials: M30 grade of concrete, Fe 415 grade HYSD bars Sketch the reinforcement diagram | CO2 CO3 | 16 2 |
| (OR) |  |  |  |  |
| 4. | a. | Enumerate the conditions under which Courbon’s method is applicable for design of bridges. | CO1 | 2 |
|  | b. | Design the longitudinal girder of T-beam girder bridge to suit the following data: Clear roadway =7.5m .Assume three T-beams spaced at 3m intervals. Effective span of T-beam =12m. Thickness of slab=200mm and wearing coat =80mm. Assume cross beams at 4 m intervals. M25grade of concrete and Fe 415 HYSD bars | CO2 CO3 | 16 2 |
| 5. | a. | Explain the failure modes of corbel. | CO1 | 4 |
|  | b. | A corbel attached to a 300mmx300mm RC column carries a factored load of 450kN at a distance of 220mm from the face of the column. Design the corbel using M20 concrete and Fe-415 bars. Sketch the reinforcement diagram of the corbel. | CO2 CO3 | 14 2 |
| (OR) |  |  |  |  |
| 6. | a. | Mention the condition under which a beam is termed as deep beam. Enlist the parameters that influence its behavior. | CO1 | 4 |
|  | b. | Design a Simply supported beam of width 400mm and 4m deep over a clear span of 7m. The beam rests on support of 0.9m and carries a load of 500kN/m including its self weight. Use M20 concrete and Fe 415 steel. Sketch the reinforcement diagram of the deep beam. | CO2 CO3 | 14 2 |
| 7. |  | Design a suitable reinforced concrete floor to cover a floor area of size 12mx15m.The floor is to be free from columns. Live load to be considered is 3 kN/m2. Use M20 concrete and Fe-415 steel. Assume suitable data. Sketch the reinforcement details. | CO2 CO3 | 18 2 |
| (OR) |  |  |  |  |
| 8. |  | A RC grid floor is to be designed to cover a floor area of 15 m x 18 m. The spacing of ribs in mutually perpendicular direction is 1.5 m c/c. Liveload on floor is 3.5 kN/m2. Adopt M20 and Fe 415. Analyze and design the grid floor using Rankine Grashoff method. Sketch the reinforcement details. | CO2 CO3 | 18 2 |
|  |  | Compulsory: |  |  |
| 9. |  | Design a conical roof for a hall of 12 m diameter and rise of 4m. Assume the incidental live load and finishes on the dome as 2 kN/m2 and 1 kN/m2. Use M20 concrete and Fe-415 steel. Sketch the reinforcement diagram. | CO2 CO3 | 18 2 |