Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14CE2030 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ADVANCED STRUCTURAL ANALYSIS | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | State Eddy’s theorem. | CO1 | 2 |
|  | b. | A two hinged parabolic arch of span 25m and central rise 4m carries a uniformly distributed load of 16kN/m over the left half of the span. Determine the position and value of maximum bending moment. Also find the normal thrust and radial shear force at the section. Assume that the moment of inertia at a section varies as secant of the inclination at the section. | CO3 | 18 |
| (OR) |  |  |  |  |
| 2. | a. | Which of the two arches, viz. circular and parabolic is preferable to carry a uniformly distributed load? Why? | CO1 | 4 |
|  | b. | A circular (three hinged) arch of span 25 m with a central rise of 5 m is hinged at the crown and the end supports. It carries a point load of 100 kN at 6 m from the left support. Calculate - The reaction at the supports and - Moment at 5 m from the left support. | CO3 | 16 |
| 3. | a. | What is the true shape of cable structures? | CO1 | 2 |
|  | b. | A three hinged stiffening girder of a suspension bridge of 100 m span subjected to two point loads 10kN each placed at 20 m and 40 m respectively from the left hand hinge. Determine the bending moment and shear force in the girder at section 30m from each end. Also determine maximum tension in the cable which has central dip of 10m. | CO3 | 18 |
| (OR) |  |  |  |  |
| 4. | a. | Briefly explain cable over a guide pulley. | CO1 | 4 |
|  | b. | A suspension bridge of 75m horizontal span and central dip 6m has a stiffening girder hinged at both the ends. The dead load transmitted to the cable including its own weight is 1500 kN. The girder carries a live load of 30kN/m uniformly distributed over the left half of the span. Assuming the girder to be rigid, calculate the shear force and bending moment in the girder at 20m from the left support. Also calculate the maximum tension in the cable. | CO3 | 16 |
| 5. |  | Analyze the Space truss shown in figure and determine the forces in the member of the truss. | CO3 | 20 |
| (OR) |  |  |  |  |
| 6. |  | Using the method of tension coefficients, analyse the plane truss in figure and find the member forces. | CO3 | 20 |
| 7. | a. | Differentiate between determinate and indeterminate structures. | CO2 | 3 |
|  | b. | A continuous beam ABC is fixed at A and has roller supports at B and C. AB = 5 m and BC = 3 m. It is subjected to udl of intensity 15 kN/m throughout the span. Analyse the beam using flexibility matrix method. EI is constant. | CO2, CO4 | 17 |
| (OR) |  |  |  |  |
| 8. | a. | Write the element flexibility matrix (α) for a truss member and for a beam element. | CO2 | 3 |
|  | b. | A cantilever of length 15 m is subjected to a single concentrated load of 50 kN at the middle of the span. Find the deflection at the free end using flexibility matrix method. EI is uniform throughout. | CO2, CO4 | 17 |
|  |  | Compulsory: |  |  |
| 9. | a. | How are the basic equations of stiffness matrix obtained? | CO2 | 3 |
|  | b. | Analyse the continuous beam ABC shown in figure by the flexibility method and draw the bending moment diagram. | CO2, CO4 | 17 |