Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14CE2032 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # BASICS OF DYNAMICS AND ASEISMIC DESIGN | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | A car moving at a speed of 100km/hr is a static or dynamic load. | CO1 | 1 |
|  | b. | Mention the relation between cyclic and circular natural frequencies. | CO1 | 1 |
|  | c. | Differentiate springs in series and parallel. | CO1 | 4 |
|  | d. | A simply supported rectangular beam has a span of 1.5m. It is 120mm wide and 12mm deep. It is connected at mid span of a beam by means of a linear spring having a stiffness of 100kg/cm and a mass of 300kg is attached to the other end of spring. Determine the natural frequency of the system. E=2.1 x 106 kg/cm2. | CO1 | 14 |
| (OR) |  |  |  |  |
| 2 | a. | Solve the equation of morion of a damped single degree of freedom and explain how the discriminant part affects the response of the system. | CO1 | 6 |
|  | b. | A SDOF system consists of a mass of 250kg and a spring stiffness of 180kN/m. By testing it was found that a force of 70 N produces a relative velocity 8 cm/s. Find a) Damping ratio b) logarithmic decrement, c) ratio of two consecutive amplitudes. | CO1 | 14 |
| 3. | a. | Mention the relation between logarithmic decrement and damping ratio. | CO1 | 2 |
|  | b. | The displacement in a system reduces from 1 to 0.5 in 10 cycles, in another system displacement reduces from 1 to 0.5 in 8 cycles. Based on logarithmic decrement discuss the behavior of the system. | CO1 | 4 |
|  | c. | A generator of 0.8 ton weight is placed on a concrete plank of width 450mm and length 2 m and thickness 100mm. Find the static and dynamic deflection of generator running at 900 rpm. Use M20 concrete. Assume damping ratio as 5% of critical. | CO1 | 14 |
| (OR) |  |  |  |  |
| 4. | a. | Differentiate steady state and transient vibration. | CO1 | 3 |
|  | b. | A column of length 3.0 m is fixed at one end and hinged at the other end. E= 2 x 105 N/mm2, I= 1.5 x 106 mm4. Determine the stiffness of the column. | CO1 | 3 |
|  | c. | Determine the natural frequency and mode shapes of the two degree of freedom system shown in figure 2m m 2k k | CO2 | 14 |
| 5. | a. | Draw the response spectrum plot. For a building resting on hard rock and time period of 0.6 sec, what will be Sa/g value? | CO2 | 4 |
|  | b. | Differentiate focus and epicenter. | CO2 | 2 |
|  | c. | Briefly discuss the plate tectonic theory. | CO2 | 14 |
| (OR) |  |  |  |  |
| 6. | a. | Discuss of the concept of bracings and the types of bracing. | CO3 | 6 |
|  | b. | Describe briefly the different seismic waves. | CO2 | 14 |
| 7. | a. | Under what circumstances do we design a structural control system? | CO3 | 4 |
|  | b. | Enlist the various structural control methods. Discuss the advantages of the various sytems mentioning case study buidings for each system. | CO3 | 16 |
| (OR) |  |  |  |  |
| 8. | a. | Discuss the design methodology for Aseismic design with respect to the architectural consideration as per IS codal provision. | CO2 | 10 |
|  | b. | Explain briefly how ductile detailing is done as per IS codes. | CO2 | 10 |
|  |  | Compulsory: |  |  |
| 9 |  | A three storey RC school building is located in Delhi with the following data Plan dimensions - 8 x 6 m Storey height - 3.5m Weight of I, II floors- 1500kN Weight of terrace – 800kN The structure is resting on hard rock. Determine the total base shear and lateral loads at each floor level for 5% of damping using seismic coefficient method. | CO2 | 20 |