Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : 14CE2037 |  | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : ADVANCED DESIGN OF STEEL STRUCTURES |  | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Differentiate between Flexible,Rigid and Semi Rigid connections. | CO1 | 6 |
|  | b. | An ISLB 300 carrying udl of 50kN/m has an effective span of 8m.This is to be connected to the web of girder ISMB 450. Design the framed connection using 20mm black bolts. | CO2 | 14 |
| (OR) |  |  |  |  |
| 2. | a. | Draw the typical sketches to show the following beam column connections a) Framed connection b) Unstiffened seated connection c) Stiffened seated connection | CO1 | 6 |
|  | b. | Design a stiffened seat angle for a reaction of 250 kN from a beam of ISMB 400 using M20 bolts of grade 4.6.The beam has to be connected to ISHB 200 column.Assume Fe410 grade steel. | CO2 | 14 |
| 3. | a. | List the advantages of steel as a structural material. | CO1 | 5 |
|  | b. | Design a stiffened welded seat connection to connect the ISMB 500 transferring a load of 260kN to an ISHB 300@577N/m. | CO2 | 15 |
| (OR) |  |  |  |  |
| 4. | a. | Briefly explain the different types of welded connection | CO1 | 5 |
|  | b. | An ISMB 400 transfers an end reaction of 160kN to the flange of an ISHB 300@ 577N/m.Design an unstiffened welded seat connection. | CO2 | 15 |
| 5. | a. | Design a channel section purlin for a roof truss having the following data Span of the truss = 6.0m Spacing of truss =3.0m c/c Inclination of roof = 30o Spacing of purlins = 2.0m c/c Wind Pressure = 1.5kN/m2 Roof coverage = A.C.Sheets weighing 200N/m2 | CO2 | 16 |
|  | b. | Summerise on types and failures of shear connectors. | CO3 | 4 |
| (OR) |  |  |  |  |
| 6. | a. | Design a rafter bracing of fink type roofing system for the following data. Length of the bracing = 9.8m Design tensile force = 13.8kN Connect the bracing system eith truss member using 2no of 16mm diameter bolts. | CO2 | 12 |
|  | b. | Ennumerate on different bracings of Lattice tower. | CO2 | 8 |
| 7. |  | A 60 m high microwave antenna lattice tower is to be built near Agra where the terrain at the site is nearly a level ground with terrain of category 2. The diameter of the Hemi-spherical antenna disc, fixed at the top is 3 m. The width of the tower at the top has to be 3 m. Select a suitable configuration for the tower and calculate the lateral load for the following data: Weight of antenna & fixtures : 9 kN Weight of platform at top : 0.82 kN/m2 Weight of railing at top : 0.30 kN/m2 Weight of ladder and the cage : 0.65 kN/m Weight of Miscellaneous items: 2.5 kN (such as beacon, lights, lightening cables etc.) Assume relevant data if required. | CO2 | 20 |
| (OR) |  |  |  |  |
| 8. | a. | Summarise on different types of steel chimneys. | CO2 | 10 |
|  | b. | Ennumerate of design loads and load combination of industrial chimney. | CO1 | 10 |
|  |  | Compulsory: |  |  |
| 9. |  | Design a simply supported composite beam with 15m span and spaced at 3m. The thickness of the slab is 100 mm. The floor is to carry an imposed load of 3.5 kN/m2, floor finish load of 0.5 kN/m2 and partition load of 1kN/m2. Take construction load as 0.75 kN/m2. fck = 25 N/mm2 and fy= 250 N/mm2. Assume relevant data if required. | CO3 | 20 |