Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14CE3005 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # STRUCTURAL DYNAMICS | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Develop the effective stiffness of the combined spring system shown in figure and write the equation of motion for the spring mass system. M k k k | CO1 | 6 |
|  | b. | A free vibration test is conducted on an empty elevated water tank. A cable attached to the tank applies a lateral horizontal force of 73 kN and pulls the tank horizontally by 50mm. The cable is suddenly cut and the resulting free vibration is recorded. At the end of four complete cycles, the time is 2.0 sec and the amplitude is 25mm compute the following: a)damping ratio b)natural period of undamped vibration c) stiffness d)weight e)damping coefficient f)number of cycles required for the displacement amplitude to decrease to 5mm. | CO1 | 14 |
| (OR) |  |  |  |  |
| 2. | a. | Using Duhamel’s Integral find the response with zero initial conditions for the forced vibration phase of the SDOF system shown | CO1 | 10 |
|  | b. | A machine, having a mass of 250 kg is supported by springs of total stiffness k = 21.5 kN/m. assuming that the damping ratio is 0.15. The machine incorporates a piston whose mass is 5 kg and it has a stroke of 200 mm. for operations at 1500 rpm. Determine the dynamic amplitude of the machine. | CO1 | 10 |
| 3. | a. | Determine the dynamic response of the tower, subjected to a blast loading. The idealization of the structure and the blast loading are shown. | CO1 | 10 |
|  | b. | For a two storey shear building with the following data determine a) the normalized modal shapes of vibration, b) verify the orthogonality conditions between the modes. m1= 20000 kg, m2 = 11500 kg, a11= 1.00, a21= 1.263, a12 = 1.00 a22 = -1.629. | CO1 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | A two storey building having a floor weight 1200kN, 600kN, for the first and second floor respectively. The height of each floor is 3m and EI = 30x1012Nmm2 . Analyze the natural frequencies and mode shapes. | CO2 | 10 |
|  | b. | Justify the statement “ normal modes are orthogonal” | CO2 | 10 |
| 5. | a. | Determine the first two frequencies of a Cantilever beam by Rayleigh-Ritz method by assuming | CO2 | 10 |
|  | b. | Develop the equation of motion for undamped forced vibration of a 2 DOF system and hence explain the concept of vibration absorber | CO2 | 10 |
| (OR) |  |  |  |  |
| 6. |  | For the multistory building shown in fig. Obtain frequencies and modes of vibration using Stodolla’s method. Assume m = 4 x 104 kg, k= 5 x 104kN/cm. m/2 2m 2m 2k 2k 2k 2k 2k 2k | CO2 | 20 |
| 7. | a. | Obtain the expression for a uniform beam subjected to free flexural vibration and hence obtain the first three natural frequencies and mode shapes of a simply supported beam subjected to free flexural vibrations. Sketch the mode shapes. | CO1 | 10 |
|  | b. | A simply supported beam of span 8m is subjected to a concentrated force of 700 N applied suddenly at a point 2m from the left end. Mass of the beam is 750 kg/m & EI = 30 x 106 Nm2. Determine the response by considering first two modes only. | CO1 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | A simply supported beam of mass” ”per unit length, length “L” and flexural rigidity “ EI” is carrying a concentrated mass “ M” at the middle span of the beam. Determine its fundamental frequency by Rayleigh’s method. | CO2 | 10 |
|  | b. | Determine the first two natural frequencies of a uniform cantilever beam by Rayleigh-ritz method. Assume ø(x)= C1x2 +C2x3 Compare the fundamental frequency with that of exact solution. | CO2 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | Using Mode superposition method, find the response of a two degree of freedom system whose Mass and Stiffness Matrices are given by the following. and The forcing function Sin 10 t. The system starts at rest. | CO2 | 10 |
|  | b. | Explain the step by step procedure for the solution of equilibrium equation in dynamic analysis using Central Difference method. | CO2 | 10 |