Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14CE3006 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # FINITE ELEMENT METHODS IN ENGINEERING | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Find the deflection at the centre of a fixed beam subjected to UDL throughout its length as in the fig.1 using point collocation method. Use the trial function as y= a(x5-2lx4+l2x3). Also find the maximum deflection at the point C and compare with the exact solution. Fig.1 | CO1 | 5 |
|  | b. | Discuss in detail about the various steps involved in FEM. | CO1 | 15 |
| (OR) |  |  |  |  |
| 2. | a. | List the applications of finite element in various engineering fields. Discuss about their advantages. | CO1 | 5 |
|  | b. | Using a three term polynomial trial function, determine the deflection at the center of a simply supported beam of span ‘l’ subjected to uniformly distributed load throughout its length as in fig.2. Use Rayleigh Ritz method. Also, compare the deflection with the value obtained from a single term trigonometric trial function. Fig. 2 | CO1 | 15 |
| 3. | a. | What is shape function? Mention their properties. | CO1 | 4 |
|  | b. | Calculate the deflection and stress for the given tapered bar shown in the fig.3 by converting it as a stepped bar with two elements. Take E = 2x105 N/mm2 Fig. 3 | CO1 | 16 |
| (OR) |  |  |  |  |
| 4. | a. | Define Natural coordinate system with suitable diagram. | CO1 | 2 |
|  | b. | For the beam and loading shown in the fig.4, calculate the rotations at B and C. Take E = 210 GPa; I = 6 x 106 mm4. Fig.4 | CO1 | 18 |
| 5. | a. | Differentiate linear triangular element and bilinear rectangular element with suitable diagram and interpolation polynomial equation. | CO1 | 4 |
|  | b. | A rectangular element with four nodes i(5, 3), j(8, 3), k(8, 5) and m(5, 5) is shown in fig.5. The nodal values are ɸi= 420C, ɸj= 540C, ɸk= 560C and ɸm= 460C. Find the temperature at the points P(6, 4) and Q(7, 4.5) Fig.5 | CO1 | 16 |
| (OR) |  |  |  |  |
| 6. |  | Derive the shape function for a Constant Strain Triangular element and check for unity. | CO1 | 20 |
| 7. |  | Derive the shape function for a 1-D quadratic element with a suitable diagram. | CO1 | 20 |
| (OR) |  |  |  |  |
| 8. | a. | What is higher order element? Discuss their advantages with an example. | CO1 | 4 |
|  | b. | Determine the shape functions of a eight noded rectangular element. | CO1 | 16 |
|  |  | Compulsory: |  |  |
| 9. |  | A furnace wall is made of three layers, thermal conductivity of the inner layer, middle layer and the outer layer are 8.5 W/mk, 0.25 W/mk and 0.08 W/mk respectively. The respective thickness of the inner, middle and outer layer are 25 cm, 5 cm and 3 cm respectively. The inside temperature of the wall is 600 °C and outside of the wall is exposed to atmospheric air at 30 °C with heat transfer coefficient of 45 W/m2k. Determine the nodal temperatures. | CO1 | 20 |