Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14CE3010 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ADVANCED BRIDGE ENGINEERING | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. |  | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | Design a RCC slab bridge for a National Highway using the following data: Carriage width = 7.5m Foot path = 1m on either side Clear span = 6.5m Wearing coat = 125mm Width of bearing = 0.5m Materials = M40 and Fe 500 Loading = IRC class AA tracked | CO1 | 20 |
| (OR) |  |  |  |  |
| 2. |  | Write the step by step design procedure for the following types of bridges? a) Bowstring girder bridge b) Arch bridge | CO1 | 20 |
| 3. |  | Design a post tensioned pre-stressed concrete slab bridge deck for a National highway crossing to suit the following data: Clear span = 10m Width of bearing = 300mm Clear width of roadways = 7.5m Foot path = 1m on each side Kerb = 600mm wide Thickness and wearing coat = 125mm Type of structure = Class I M50 grade concrete and 7mm dia high tensile wires with an ultimate tensile strength of 1700 N/mm2 housed in cables with 12 wires and anchored by Freyssinet anchorage of 150mm dia. For supplementary reinforcement, adopt Fe500 grade steel. Compressive strength at transfer is 50 N/mm2 and loss ratio is 0.8. | CO1 | 20 |
| (OR) |  |  |  |  |
| 4. |  | Discuss the major causes of Prestressed concrete bridge failures and suggest suitable remedial measures to avoid the failures. | CO2 | 20 |
| 5. |  | Design the welded plate girder bridge for the following data? Span - 10m Loading as per standards Steel of grade Fe 410 Assume the relevant design details | CO1 | 20 |
| (OR) |  |  |  |  |
| 6. |  | The foundation for sub structure of a bridge consists of 16 piles to carry a total load of 7,000 kN The piles are spaced at 1m c/c. They are driven through soft ground to a hard stratum available at a depth of 10m. Design the pile foundation using M50 concrete and Fe 500 steel. | CO1 | 20 |
| 7. |  | Verify the stability of the Rectangular pier for the following details. Width of the pier – 2m Ht of the pier – 12m C/C of bearings on either side – 1m High flood level – 1m below bearing level Span of the bridge – 16m Live load – IRC AA No of Longitudinal girders – 3, Size of the girder 0.3x1.2m Thickness of the slab – 200mm Assume the relevant design details | CO2 | 20 |
| (OR) |  |  |  |  |
| 8. |  | Explain in detail about the different types of Bearings used in bridge construction with neat sketches? | CO2 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | Discuss the various methods usually adopted in maintenance and rehabilitation of the existing RCC & Steel bridges. | CO2 | 20 |