Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14CE3013 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # DESIGN OF STRUCTURES FOR DYNAMIC LOAD | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Compare the behaviour of steel and concrete under cyclic load. | CO1 | 10 |
|  | b. | Discuss the difference in loading pattern of blast, earthquake and wind loads. | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Enumerate the step by step procedure for push over analysis. | CO2 | 10 |
|  | b. | Discuss the precautions to be taken in the design of life line structures. | CO1 | 10 |
| 3. |  | Explain how towers and tunnels are designed to resist earthquakes. | CO1 | 20 |
| (OR) |  |  |  |  |
| 4. | a. | Enumerate the step by step procedure for push over analysis. | CO2 | 12 |
|  | b. | Briefly discuss progressive collapse method for design against blast. | CO1 | 8 |
| 5. |  | A building of size 12 x 12 m in plan and 36m in height is located in Chennai at a distance of 200 m from the sea face. Determine the distribution of Wind pressure along the height of the building. Assume suitable data. | CO2 | 20 |
| (OR) |  |  |  |  |
| 6. | a. | Discuss the limitations of height of buildings for design earthquake and wind forces. For design of wind forces how the nodal loads are calculated. | CO1 | 10 |
|  | b. | Briefly discuss on gust factor method for design against wind. | CO2 | 10 |
| 7. |  | A 100 m tall RC cylindrical chimney of uniform cross section (A = 8 m2, I = 80 m4) has weight of 25 kN/m3 and E = 4X104N/mm2. Determine the base moment and shear under earthquake conditions. The structure is located in seismic zone V and is supported on raft foundation. Take damping factor as 5% and importance factor as 2. | CO2 | 20 |
| (OR) |  |  |  |  |
| 8. | a. | Explain the difference in the principles of bracings and base isolator. | CO3 | 10 |
|  | b. | Discuss the advantages of semiactive control over passive control systems. | CO3 | 10 |
|  |  | Compulsory: |  |  |
| 9. |  | Discuss in detail the methodology for System Identification. | CO2 | 20 |