Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14CE3018 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # DESIGN OF SUBSTRUCTURES | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

(Use of IS;2911(Part 1/Sec. 1), IS:6403 and IS;456 Permitted)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Define the term, “significant depth of exploration” and list the empirical guidelines for determining the depth of exploration | CO1 | 5 |
|  | b. | Bring out the different objectives of site investigation | CO1 | 3 |
|  | c. | Define the term ‘Foundation’ and explain briefly the Classification of foundation with neat sketches | CO1 | 7 |
|  | d. | Explain any one NDT technique of soil investigation | CO1 | 5 |
| (OR) |  |  |  |  |
| 2. | a. | Enumerate the steps involved in subsoil exploration | CO1 | 5 |
|  | b. | Explain disturbed and undisturbed soil samples | CO1 | 4 |
|  | c. | State the key steps to be performed by a foundation engineer in arriving at the most appropriate foundation for a given structure at a given site | CO1 | 5 |
|  | d. | Explain with neat sketch how the Standard Penetration Test is carried out in the field | CO1 | 6 |
| 3. | a. | List the criteria for proportioning of a cantilever footing. | CO2 | 5 |
|  | b. | What is the basic principle of a floating type mat foundations? | CO2 | 5 |
|  | c. | Estimate the dimension of a square footing to carry a load of 2000kN from a column. The depth of foundation is to be kept at 2 m below the ground level. A factor of safety of 3 is required against shear failure. The footing is resting in sand with an average corrected N value of 20 as estimated from soil borings. Unit weight of soil is 19 kN/m3 The ground water table is at a larger depth. Estimate the width of square footing | CO2 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | Explain Terzaghi’s bearing capacity theory | CO2 | 5 |
|  | b. | List the site conditions for various types of combined footings | CO2 | 5 |
|  | c. | A building has to be supported on a R.C. raft foundation of dimensions 14 m X2 m. The subsoil is clay having an average unconfined compressive strength of 30 kN/m2 . The pressure on the soil due to weight of the building and the loads that it carry will be 150kN / m 2 at the base of the raft. If the unit weight of the excavated soil is 19 kN/m3 , at what depth should the bottom of the raft be placed to provide a factor of safety of 3 against shear failure? Use Skempton's bearing capacity formula. | CO2 | 10 |
| 5. | a. | Explain negative skin friction. | CO2 | 5 |
|  | b. | Write down the precautions to avoid tilts and shifts | CO2 | 5 |
|  | c. | A concrete pile of 45cm diameter is driven through a system of layered cohesive soils. The length of the pile is 16m. The following data are available. The water table is close to the ground surface. Top layer: 1 Soft clay, thickness = 8m, Cu = 30kN/m2 α = 0.9 Layer 2 Medium stiff, clay thickness = 6m, Cu = 50kN/m2 α =0.75 Layer : 3 Stiff stratum extends to great depth, Cu.105kN/m2 α =0.5 . Compute Qu and Qa with F.S. = 2.5 | CO2 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Enumerate the various classification of piles | CO2 | 5 |
|  | b. | Enumerate the methods of determining the settlement of a pile group | CO2 | 4 |
|  | c. | A RC pile weighing 35kN( inclusive of self weight and dolly) is driven by a drop hammer weighing 45 kN and having an effective fall of 1 m. Average set per blow is 16mm. The total elastic compression observed is 18mm. Determine the ultimate load allowable load of the pile. Take the coefficient of restitution as 0.25 and factor of safety as 2 | CO2 | 5 |
|  | d. | A square group of 9 piles was driven into soft clay extending to a large depth. The diameter and length of piles was 30cm and 9m respectively. If the unconfined compression strength of clay is 90 kN/m2 and the pile spacing is 90cm center to center, what is capacity of the group? Assume a factor of safety of 2.5 and adhesion factor of 0.75 | CO2 | 6 |
| 7. | a. | Explain Terzaghi’s method of analysis of well foundation | CO2 | 6 |
|  | b. | Bring out the design criteria for the design of foundation for Impact machines | CO2 | 6 |
|  | c. | The resonant frequency of a block foundation excited by an oscillator is observed as 20 Hz. The amplitude of vibration at resonance is 1mm. The magnitude of dynamic force at 20 Hz is 5kN. If the total weight of block and oscillator is 20 kN, Calculate the damping factor associated with it. | CO2 | 8 |
| (OR) |  |  |  |  |
| 8. | a. | Explain the various vibration isolation techniques in machine foundation | CO2 | 6 |
|  | b. | Discuss the design requirements for foundation of impact type of machine | CO2 | 6 |
|  | c. | A foundation block of weight 30 kN rests on a soil for which the stiffness is 25000 kN/m. Find the natural frequency, natural period, natural circular frequency and the amplitude of vertical displacement of the machine. Take the damping factor as 0.50. | CO2 | 8 |
|  |  | Compulsory: |  |  |
| 9. |  | Discuss in detail the application areas of reinforced earth with neat sketches | CO1 | 20 |