Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14CE3019 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # DESIGN OF COMPOSITE STRUCTURES | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | Explain various advantages/disadvantages and applications of composite materials with examples as compared to other conventional construction materials. | CO1 | 20 |
| (OR) |  |  |  |  |
| 2. |  | Describe the ultimate load theory and moment of resistance of composite beams as per IS & Eurocode. | CO1 | 20 |
| 3. |  | A composite floor slab is supported on three span continuous composite beam spaced at 4m centres. The effective length of each span is 5m. Thickness of composite slab is 150mm. The floor has to carry an imposed load of 4kN/m2, Partition load 1 kN/m2, Floor finish load 1kN/m2 and Construction load of 1kN/m2. Design the Continuous beam for the given loading. | CO2 | 20 |
| (OR) |  |  |  |  |
| 4. | a. | Draw the various types of stiffeners in profiled deck sheets. | CO1 | 5 |
|  | b. | Design a profiled deck slab of 5m span. The thickness of the slab is 125mm. The floor is to carry a imposed load of 5kN/m2, partition load of 1kN/m2 and floor finish load of 1kN/m2. Take Construction load as 1kN/m2. fck= 25N/mm2. Moment of Inertia of the section – 0.75x106 mm4, Plastic Moment of Resistance – 10 kNm, Area of cross section – 1185mm2, Depth of the profile – 75mm | CO2 | 15 |
| 5. |  | Design the fully encased composite column subjected to axial load and uniaxial bending moment for the following data. Size of column - 500x500mm Axial load - 600kN Bending moment - 250kNm Grade of concrete - M30 Structural steel - ISHB 300 | CO2 | 20 |
| (OR) |  |  |  |  |
| 6. | a. | Calculate the load carrying capacity of shear connector having 16mm dia and 150m height as per IS and BS provision. | CO2 | 5 |
|  | b. | Design the composite column for the following data. Size of column - 300x500mm Axial load - 1000kN Grade of concrete - M25 Structural steel - ISHB 250 | CO2 | 15 |
| 7. |  | Write the interaction between shear connector and the connecting slab? | CO3 | 20 |
| (OR) |  |  |  |  |
| 8. | a. | Explain the load test to be conducted to check the load carrying capacity of shear connector? | CO3 | 15 |
|  | b. | Draw the different types of composite beam column joints. | CO1 | 5 |
|  |  | Compulsory: |  |  |
| 9. |  | - Discuss the lateral load behaviour of Composite structures? | CO3 | 20 |