Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14CE3021 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # PRESTRESSED CONCRETE STRUCTURES | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | A pre-tensioned concrete beam of 100 mm wide and 300 mm deep is prestressed by straight wires carrying an initial force of 150 kN at an eccentricity of 40 mm. The modulus of elasticity of steel and concrete are 210 and 35 kN/mm2 respectively. Estimate the percentage loss of stress in steel due to elastic deformation of concrete if the area of steel wire is 200 mm2. | CO1 | 10 |
|  | b. | A concrete beam of rectangular section, 100mm wide and 300mm deep is prestressed by 5 wires of 7 mm diameter located with an eccentricity of 50mm, the initial stress in wires are 1200 N/mm2. Estimate the loss of stress in steel due to creep of concrete. Creep coefficient 1.6. | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. |  | A rectangular concrete beam of c/s 200x300 mm isprestressed by 15 wires of 5 mm diameter located 65 mm from the bottom of the beam and 3 wires of 5 mm diameter at 25 mm from the top, prestress in the steel is 840 N/mm2. Calculate the stresses at the extreme fibers of mid-section, when the beam is supporting its own weight over a span of 6m. Imposed load on the beam is 6 kN/m. | CO3 | 20 |
| 3. |  | Design a post tensioned beam of span 20 m for a live load of 18kN/m. Strength of concrete at transfer is 35 N/mm2 and the wires are initially stretched to 1200 MPa. M40 grade concrete and 5mm dia steel wire of tensile strength 1800 N/mm2 are to be used. | CO2 | 20 |
| (OR) |  |  |  |  |
| 4. |  | Calculate the limit state of moment of resistance at the mid-span section of a slab for the following data. Depth of the section – 440 mm, Area of prestress 2770 mm2, Grade of concrete M40, Tensile strength of steel – 1520 N/mm2, Prestressing force – 3100kN, Young’s modulus of steel – 2.1x105 N/mm2 | CO1 | 20 |
| 5. |  | A prestressed concrete beam of 300x600mm rectangular section is prestressed by means of 2 cables each consisting of 12 number of 5 mm dia, HTS wires with an eccentricity of 175 mm throughout its span of 10meters. Grade of concrete – M45. Calculate the deflection at midspan at the time of transfer of prestress, if the initial prestress in the HTS wires are 1150 N/mm2. Assume the cable is straight. | CO1 | 20 |
| (OR) |  |  |  |  |
| 6. |  | A rectangular beam of c/s 350 x 850mm is subjected to an effective prestressing force of 2000 kN acting at the centroid of the section. fci=fck=35 N/mm2, fy=415 N/mm2 for ties and loss of prestress15%, the cables pass through a steel plate symmetrically in an area of 200 x300mm. Design the anchor plate and the reinforcements for bursting and spalling tensile forces by IS Method. | CO2 | 20 |
| 7. |  | A continuous prestressed concrete beam of uniform section has cable profile as shown in Figure The magnitude of the prestressing force is 1500kN - Locate the line of pressure (C-line) in the concrete due to prestress alone not considering the dead load of the beam. - What is the secondary moment at section B. - Determine the maximum stress at the mid span section of BC due to dead load and prestressing force if cross section is a rectangle of breadth 300mm and depth 800mm. | CO3 | 20 |
| (OR) |  |  |  |  |
| 8. |  | Design a composite T beam using the following data: Span = 8m Spacing of beams = 1m Expected loss of prestress = 20% Live load = 40kN/m2 Grade of concrete = M40 Take fci = fck | CO2 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | Design a precast post tensioned column to carry an axial load of 200 kN and bending moment of 15 kNm, the length of the column is 4 m with both ends hinged. Loss of prestress 20%. Permissible compressive stress in concrete 12 N/mm2 and tensile stress in 3 mm dia wire is 1200 N/mm2, size of the column 300x300 mm. | CO2 | 20 |