Reg.No. ____________

End Semester Examination – Apr/May – 2018

| Code : | 14CS2005 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | COMPUTER ARCHITECTURE | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Describe the structural and functional components of computer with its neat diagram. | CO1 | 10 |
|  | b. | Sketch the John Von Neumann’s top level view of computer and state diagram for instruction cycle. | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Illustrate types of multiple interrupts with suitable examples. | CO1 | 10 |
|  | b. | Consider the following: Main memory size : 32 MB Cache memory size : 8 KB Block size : 4Bytes Mapping Function : Direct Mapping - Calculate the address bits required for main memory - Calculate the number of cache lines in cache memory - Calculate the tag bit size | CO3 | 10 |
| 3. |  | Consider the following for bit positions for 8 bit data: 8 Bit Data: 1000 1110 for the storage. Calculate the check bit for above data bits. While reading the same data bits from storage is following: 1000 0110 Calculate the check bit for the data that is read from storage. Calculate the syndrome word from above 2 check bits and find where the error is? ( Explain with step by step procedure) | CO3 | 20 |
| (OR) |  |  |  |  |
| 4. |  | Describe the block diagram of an I/O Module and elaborate its three techniques with neat flowchart. | CO2 | 20 |
| 5. |  | Explain Booth’s algorithm of signed multiplication with its flowchart and Step by step procedure for -4×3. | CO1 | 20 |
| (OR) |  |  |  |  |
| 6. |  | Recognize the following Equations for Addressing modes and brief the respective addressing modes. Also illustrate the diagram for each addressing mode. i) EA = (R) ii) EA = (A) iii) EA = A iv)EA=R | CO3 | 20 |
| 7. |  | Sketch the timing diagram of 14 clock pulse with 4 stages: Fetch Instruction (FI),Decode Instruction (DI),Calculate operands(CO), Execute Instruction (EI) for following scenario - With no branches - If at the time 7, instruction 3 is in execute stage and executes a branch to instruction 15. | CO3 | 20 |
| (OR) |  |  |  |  |
| 8. |  | Classify the pipeline hazards and illustrate with suitable timing diagram examples. | CO3 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | Discuss the microinstruction sequencing and its execution in detail with suitable diagrams. | CO3 | 20 |