Reg.No. ____________

End Semester Examination – Apr/May – 2018

| Code : | 14CS2005 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | COMPUTER ARCHITECTURE | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | The following diagram shows the initial positions of memory and CPU registers. An addition program is stored in memory locations 300, 301 and 302. The values are stored in memory locations 940 and 941. Explain the steps during program execution and show the final values in all these memory locations after program execution. Note : 0001 – 1 ; 0010 – 2; 0101 – 5 | CO1 | 15 |
|  | b. | Contrast the dedicated and multiplexed bus types. | CO1 | 5 |
| (OR) |  |  |  |  |
| 2. | a. | Discuss any two mechanisms to handle simultaneously occurring multiple interrupts with suitable examples. | CO1 | 10 |
|  | b. | Discuss the procedure of associative mapping technique that maps each main memory block into any cache memory line, with hardware configuration diagram. | CO3 | 10 |
| 3. | a. | Consider that an 8-bit data word 11000010 is stored in memory. Using the Hamming code, calculate the check bits to be stored in memory along with the data word. Apply the single bit error correction technique to detect the error in this word if the word is read out as 10000010. | CO3 | 15 |
|  | b. | Discuss the various cache replacement policies with appropriate examples. | CO1 | 5 |
| (OR) |  |  |  |  |
| 4. | a. | The Direct Memory Access technique requires the least involvement of the processor than programmed I/O and interrupt driven I/O techniques, in any input output process. Justify the above statement with suitable examples. | CO2 | 15 |
|  | b. | Discuss Interrupt Driven I/O mechanism which deals the I/O operation in a program execution. | CO2 | 5 |
| 5. | a. | Evaluate the capability of Booth’s algorithm to multiply the negative numbers -7 and -3. | CO1 | 15 |
|  | b. | Differentiate logical right shift and arithmetic shift for the following example. 1; 0; 0; 1; 0; 0; 1; 1. | CO2 | 5 |
| (OR) |  |  |  |  |
| 6. | a. | Consider the following instructions: EA = (A); 1. EA = A + (R); 2. EA = R; 3. Identify the addressing modes used in the above instructions and explain the addressing modes. | CO3 | 15 |
|  | b. | How will you identify which address instruction format is suitable for high performance in program execution? | CO2 | 5 |
| 7. | a. | Identify the hazard in the above mentioned instruction pipeline. Explain that hazard in detail and propose solutions for the same. | CO2 | 15 |
|  | b. | Draw and explain the instruction cycle state diagram which has indirect cycle. | CO3 | 5 |
| (OR) |  |  |  |  |
| 8. | a. | Elaborate the control hazards with suitable programming examples and discuss the various branch prediction techniques to reduce the branch penalty. | CO3 | 15 |
|  | b. | Outline the various flags and their functions available in the PSW register. | CO2 | 5 |
|  |  | Compulsory: |  |  |
| 9. |  | Discuss the microinstruction sequencing and its execution in detail with suitable diagrams. | CO3 | 20 |