Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14CS2007 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # COMPUTER NETWORKS | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | List the advantages of circuit-switched network over a packet switched network? | CO1 | 10 |
|  | b. | Compare TDM and FDM in a circuit-switched network? | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | A caravan of ten cars travels 200km at a propagation speed of 100km/hr, beginning in front of one tollbooth, passing through a second tollbooth, and finishing just before a third toll-booth. Calculate the end-to-end delay? | CO2 | 10 |
|  | b. | Elaborate the functions of IP layers. | CO3 | 10 |
| 3. | a. | Suppose Alice, with a Web-based e-mail account, sends a message to Bob, who accesses his mail from his mail server using POP3. Discuss how the message gets from Alice’s host to Bob’s host. Be sure to list the series of application- layer protocols that are used to move the message. | CO2 | 15 |
|  | b. | Compare and contrast download-and-delete mode and the download-and-keep mode in POP3? | CO3 | 5 |
| (OR) |  |  |  |  |
| 4. | a. | Summarise the working of DNS. | CO3 | 10 |
|  | b. | Explain Web-caching with suitable diagram. | CO1 | 10 |
| 5. | a. | Explain rdt 2.0 FSM with neat sketch. Discuss the limitations of rdt2.0 | CO2 | 10 |
|  | b. | Distinguish TCP and UDP protocol. | CO3 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Define pipelined protocol. Discuss in detail with examples. | CO3 | 10 |
|  | b. | Elaborate various approaches for congestion control. | CO3 | 10 |
| 7. | a. | Given the IP address 192.168.1.0/24, devise an optimum IP addressing scheme for the following network topology having equal sized subnets. | CO2 | 10 |
|  | b. | Summarise the working of DHCP protocol. | CO1 | 10 |
| (OR) |  |  |  |  |
| 8. |  | Illustrate Distance-Vector Routing Algorithm. | CO2 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | Explain various error correction and detection techniques with suitable examples. | CO3 | 20 |