Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14EC1001 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # BASIC ELECTRONICS ENGINEERING | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Discuss the types of passive element that stores energy in the form of magnetic field. | CO1 | 6 |
|  | b. | Discuss the various types of fixed passive element that has a property to oppose the flow of current in detail. | CO1 | 14 |
| (OR) |  |  |  |  |
| 2. | a. | Explain the formation of electrons and holes in N type Semiconductor through its covalent bond structure. | CO2 | 6 |
|  | b. | Explain the various types of capacitors that uses plastic and paper as di-electric material. | CO1 | 14 |
| 3. | a. | With a neat circuit diagram explain the input and output characteristics of NPN transistor in CE configuration. | CO1 | 12 |
|  | b. | Describe the operation of PN Junction diode under forward bias and reverse bias conditions with suitable circuit diagram. | CO2 | 8 |
| (OR) |  |  |  |  |
| 4. | a. | With neat diagrams, explain the construction and operation of N-Channel MOSFET in enhancement and in depletion mode. | CO1 | 15 |
|  | b. | Show how a Zener diode is used as a voltage regulator with help of V-I characteristics curve. Show how a zener diode is used as a voltage regulator. | CO2 | 5 |
| 5. | a. | Solve using 4-variable K-map: y=∑m(0,1,2,3,12,13,14,15) | CO2 | 10 |
|  | b. | With symbol, electrical equivalent circuit diagram and its truth table, explain the working of AND and OR gate. | CO1 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Design a 1x4 Demultiplexer and explain its operation with neat logic diagram. | CO2 | 10 |
|  | b. | # Simply the following using Boolean algebra. (i)A+AB+AB’C (ii) AB+AC+BC |  | 10 |
| 7. | a. | What is the need for modulation? | CO2 | 6 |
|  | b. | Derive the expression of Amplitude modulation and its power calculation with neat waveforms. | CO3 | 14 |
| (OR) |  |  |  |  |
| 8. | a. | Explain the Super heterodyne receiver with a neat block diagram. | CO2 | 10 |
|  | b. | Draw the block diagram and explain about the Amplitude Modulation Transmitter. | CO3 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | Draw and explain the block diagram of a Radar System. | CO3 | 10 |
|  | b. | Draw the block diagram of Satellite communication system and explain the working principle of the same. | CO3 | 10 |