Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14EC2001 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # DIGITAL ELECTRONICS | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Simplify the following using K map F (a,b,c,d) = ∑ m (0,2,6,8,10) + d (4,14). | CO1 | 6 |
|  | b. | Place the equation J = f (A,B,C,D) = (A + B' +C) (A' + D) into the proper canonical form. Convert the obtained canonical form into SoP form and draw the logic diagram. | CO1 | 14 |
| (OR) |  |  |  |  |
| 2. | a. | Draw logic diagram and truth table for the Boolean expression given Q=AB'+AC | CO1 | 6 |
|  | b. | Simplify the given Boolean function using k-map F (a,b,c,d) = Σ (11,13,14,15) | CO1 | 6 |
|  | c. | Give the 8’s compliment of (562)8 ________________ Give the equivalent Excess 3 code for 7 -------------------- Give the equivalent BCD code for 17 -------------------- | CO1 | 6 |
|  | d. | Consider a four bit digital to analog converter. The analog value corresponding to digital signals of value 0000 and 0001 are 0 volts 0.0625 volts respectively. The analog value (in volts) corresponding to the digital value 1111 is _______ | CO1 | 2 |
| 3. | a. | Convert the following expression into canonical form F(A,B,C)=(A+B).(A+C) | CO1 | 4 |
|  | b. | Convert (250.5)10 to base 4. | CO1 | 6 |
|  | c. | Simplify the following equation using Quine-Mc-Cluskey method F(a,b,c,d)=Σ (0,1,2,8,10,11,14,15) | CO1 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | Design a 1 to 4 DeMUX circuit. | CO2 | 6 |
|  | b. | Design a look ahead carry generator circuit with neat diagram. | CO2 | 10 |
|  | c. | Implement the following Boolean expression with exclusive-OR and AND gates and draw the logic diagram. F=AB’CD’+A’BCD’+AB’C’D+A’BC’D | CO1 | 4 |
| 5. | a. | Design a BCD to Excess3 code converter circuit. | CO2 | 10 |
|  | b. | Implement the carry of full adder circuit using MUX | CO2 | 4 |
|  | c. | In the circuit shown in Fig 1.1, if C = 0, the expression for Y is _______. Justify the same. Fig 1.1 - Y=AB+AB - Y = A + B - Y = A+B - Y = AB | CO3 | 6 |
| (OR) |  |  |  |  |
| 6. | a. | Implement the expression (B+C)A using NOR gate | CO1 | 4 |
|  | b. | Design a MOD 3 counter with unused states using T flipflop. | CO2 | 6 |
|  | c. | Design a Parallel in Serial out shift register and explain its operation with a neat timing diagram. | CO2 | 10 |
| 7. | a. | With sequence table and diagram explain Johnson counter | CO2 | 5 |
|  | b. | Design a 3 bit synchronous binary up-down binary using T flip flop. | CO2 | 10 |
|  | c. | Design a 3 bit even parity generator circuit. | CO2 | 5 |
| (OR) |  |  |  |  |
| 8. | a. | With state diagram differentiate between Moore and Mealy Machine | CO2 | 6 |
|  | b. | Design a sequential circuit using JK flip flop for the given circuit behavior. Present State; W; Y; W; X; X; Y. Input; 0; 1; 1; 0; 1; 0. Output; 0; 0; 0; 1; 1; 0. Next State; Y; W; X; X; Y; X. | CO2 | 14 |
|  |  | Compulsory: |  |  |
| 9. | a. | Draw the timing diagram of 4 bit binary ripple counter. | CO2 | 6 |
|  | b. | Design a 3 input NAND gate using open collector TTL logic Families | CO3 | 8 |
|  | c. | Find minimum SoP for the following PoS and implement using PLA F(A,B,C,D) = π (4,6,7,8,12,13,14,15) | CO3 | 6 |