Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14EC2001 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # DIGITAL ELECTRONICS | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Simplify the following using Boolean theorems: - f(x,y,z)=(x+y)[(x’(y’+z’)]’ + (xy)’ +(xz)’ - f(A,B,C) = (A+B+C)(A’+B+C)(A’+B+C’) | CO1 | 10 |
|  | b. | Demonstrate by means of Truth table the validity of following theorem of Boolean algebra - Associative law - Demorgans law for Validity - Distributive law. | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Draw logic diagram and truth table for the Boolean expression given Q=ABC+A’B | CO1 | 10 |
|  | b. | Simplify the given Boolean function using k-map F (a,b,c,d) = Σ (1,2,3,,4,8,9,11,13,14,15) | CO1 | 10 |
| 3. | a. | Convert the following to other canonical form - F(x,y,z) =(1,3,7) - F(A,B,C,D)= (0,2,6,11,13,14) | CO1 | 10 |
|  | b. | Simplify the following equation using Quine-Mc-Cluskey method F(a,b,c,d)=Σ(0,1,2,8,10,11,14,15) | CO1 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | Design a 1 to 4 DeMUX circuit. | CO2 | 5 |
|  | b. | Design a lookaheadcarry generator circuit with neat diagram. | CO2 | 10 |
|  | c. | Implement the following function F=(CD+E) (A+B’) using NAND gates only. | CO1 | 5 |
| 5. | a. | Design a BCD to Excess3 code converter circuit. | CO2 | 10 |
|  | b. | Implement a full subtractor with two half subtractors and an OR gate. | CO2 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Prove w’x+yz’ = 0. | CO1 | 4 |
|  | b. | Design a MOD 4 counter with unused states using T flipflop. | CO2 | 6 |
|  | c. | Design a Serial in Serial out shift register and explain its operation with a neat timing diagram. | CO2 | 10 |
| 7. | a. | With sequence table and diagram explain Johnson counter. | CO2 | 5 |
|  | b. | Design a 4 bit synchronous binary up-down binary using T flip flop. | CO2 | 10 |
|  | c. | Design a 3 bit even parity generator circuit. | CO2 | 5 |
| (OR) |  |  |  |  |
| 8. | a. | With state diagram differentiate between Moore and Mealy Machine. | CO3 | 8 |
|  | b | Design BCD counter with JK flip flops. | CO3 | 12 |
|  |  | Compulsory: |  |  |
| 9. | a. | Draw the timing diagram, of 3 bit binary ripple counter. | CO2 | 6 |
|  | b. | Design a 3 input NAND gate using open collector TTL logic Families | CO3 | 8 |
|  | c. | Find minimum SoP for the following PoS and implement using PLA F(A,B,C,D)= π(1,2,3,6,8,9,10,12,14) | CO3 | 6 |