Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14EC2002 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ELECTRON DEVICES | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Calculate the temperature at which there is a 1% probability that a state 0.3 eV below the fermi energy level will not contain an electron. Assume the fermi energy for the material is 6.25 eV. | CO1 | 10 |
|  | b. | Derive the mathematical equation for hole concentration p0 in a compensated semiconductor which has Na > Nd. | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Calculate the thermal equilibrium hole concentration in silicon at T=400 K. Assume that the fermi energy is 0.27 eV above the valance band energy. The value of Nv for silicon at T=300 K is 1.04x1019 cm-3. | CO1 | 10 |
|  | b. | Derive the expression of electron and hole concentration for an extrinsic semiconductor. | CO1 | 10 |
| 3. | a. | Prove that the majority carrier hole concentration is inversely proportional to the Hall voltage. | CO1 | 10 |
|  | b. | Explain the different types of carrier generation process in detail with necessary diagrams. | CO1 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | Describe the current components in a pn diode. | CO2 | 10 |
|  | b. | Draw the band structure of an open circuited pn junction and derive the contact potential V0 across the junction. | CO2 | 10 |
| 5. |  | Explain how the charge density varies with respect to time and distance in a small slice of an n type semiconductor using Continuity equation. | CO2 | 20 |
| (OR) |  |  |  |  |
| 6. |  | Explain the operation of PN junction diode with V-I characteristics. | CO2 | 20 |
| 7. |  | Analyze the input and output characteristics of common emitter bipolar junction transistor. | CO2 | 20 |
| (OR) |  |  |  |  |
| 8. |  | Enumerate the construction and operation of Uni Junction Transistor(UJT) with necessary diagrams. List out the applications of the same. | CO2 | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | Write short notes on Light Emitting Diode. | CO2 | 10 |
|  | b. | Explain the operation and construction of Silicon Controlled Rectifier (SCR). | CO2 | 10 |