Reg.No. ____________

End Semester Examination – Apr/May – 2018

| Code : | 14EC2002 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | ELECTRON DEVICES | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Draw the equilibrium distribution of electrons and holes in intrinsic semiconductor and write the mathematical expression for concentration of electrons and holes. | CO1 | 15 |
|  | b. | Assume fermi energy level is 0.3eV above the valence band. The value of Nv for Ge is 6x1018cm-3. Calculate thermal equilibrium hole concentration (Boltzmann constant k=8.6173×10—5eVK-1, T=300K). | CO1 | 5 |
| (OR) |  |  |  |  |
| 2. | a. | Draw the energy band structure of compensated semiconductor and derive the electron concentration for n-type compensated semiconductor. | CO1 | 15 |
|  | b. | Prove n0p0 =ni2 for intrinsic semiconductor. | CO1 | 5 |
| 3. |  | Summarize the methods of carrier generation and recombination in semiconductor with necessary diagrams. | CO1 | 20 |
| (OR) |  |  |  |  |
| 4. | a. | Derive the continuity equation for holes in the body of a semiconductor as a function of time and distance. | CO2 | 10 |
|  | b. | Draw and explain the construction, operation and volt-ampere characteristics of semiconductor device which is used as a rectifier, clipper and clamper. | CO2 | 10 |
| 5. |  | Explain the hole and electron current components with respect to distance(x) in a pn junction. Also derive the expression for the total current. | CO3 | 20 |
| (OR) |  |  |  |  |
| 6. | a. | Explain the operation of NPN and PNP transistors. Also list the applications of transistor. | CO2 | 10 |
|  | b. | Plot the input and output characteristics of common base configuration of BJT. | CO2 | 10 |
| 7. |  | Explain the construction and operation of N-channel JFET with characteristics curves. | CO2 | 20 |
| (OR) |  |  |  |  |
| 8. |  | Draw the equivalent circuit of UJT and explain its operation with the help of emitter characteristics curve. | CO3 | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | Explain in detail the construction, equivalent circuit, working and characteristics of SCR. | CO3 | 15 |
|  | b. | Explain the operation of heavily doped diode which is used as voltage regulator. | CO3 | 5 |