Reg.No. ____________

End Semester Examination – April/May 2018

| # Code : | # 14EC2003 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # SIGNALS AND SYSTEMS | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Check whether given system y(t) is (a) Time-invariant (b) static (c) Linear (d) causal and justify the answer y(t) = x(t + 12) + x²(t) | CO1 | 12 |
|  | b. | Test whether the following signal is periodic or not. If the signal is periodic, calculate the fundamental period. - x(t)=2cos3πt+7cos9t - x(n)=1+ ej4πn/7 - ej2πn/5 | CO1 | 8 |
| (OR) |  |  |  |  |
| 2. | a. | An LTI system is characterized by its impulse response h[n] = {1,3,3,2}. Determine the response of the system to an input x[n] = {1,3,1,1}. Use graphical method. | CO1 | 12 |
|  | b. | x (t) t 1 -1 1 The square pulse signal x(t) is given below. Sketch each of the following signals derived from x(t) - x (3t) - x (3t+2) - x(-2t-1) - x(2(t+2)) | CO1 | 8 |
| 3. |  | Consider a stable LTI system characterized by the differential equation d2y(t)/dt2 +6dy(t)/d(t)+8y(t)=2x(t) Find - impulse response of the system - What is the response of the system if x(t)=te -2t u(t) | CO2 | 20 |
| (OR) |  |  |  |  |
| 4. | a. | Find the Fourier transform for the following signal and sketch the frequency response x(t)=e -at u(t) | CO2 | 15 |
|  | b. | Find Inverse Fourier Transform for the following signals X(jω)=πe-¦ω¦ | CO2 | 5 |
| 5. | a. | Find the impulse response and step response of a system with transfer function H(s) = 3s + 4(s+1) (s+2)2 | CO2 | 15 |
|  | b. | Find the Nyquist rate and Nyquist interval for the following signal x(t)= sin(4000t) sin(1000t) | CO2 | 5 |
| (OR) |  |  |  |  |
| 6. |  | Show that a signal x(t) can be uniquely reconstructed from its samples if its sampling frequency Fs> 2Fm. | CO2 | 20 |
| 7. |  | State and prove anyfive properties of DTFT. | CO3 | 20 |
| (OR) |  |  |  |  |
| 8. |  | Consider a LTI system which is characterized by the difference equation y(n)-(3/4)y(n-1)+(1/8)y(n-2)=2x(n). - Find the impulse response - Determine the output of the system for the input x(n)=(1/4)n u(n) | CO3 | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | Find the Z transform and ROC of the following signals - x(n)=nu(n) - x(n)=7(1/3)n u(n)-6(1/2)n u(n) | CO3 | 10 |
|  | b. | By using Long Division method, Find the inverse Z transform of X(Z)=(1+2z-1)/(1-2z-1+z-2) when i) x(n) is causal (ii) x(n) is anti-causal | CO3 | 10 |