Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14EC2006 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ELECTRONIC CIRCUITS | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | For a full wave rectifier, voltage between center tap and secondary terminal is 20V. Load resistance is 500Ω and diode forward resistance is 20Ω. Find - DC current(Idc) - DC output voltage(Vdc) - RMS Current(Irms) - Efficiency(η) - Ripple factor(Γ) | CO1 | 15 |
|  | b. | Compare half wave rectifier with full wave rectifier. | CO1 | 5 |
| (OR) |  |  |  |  |
| 2. | a. | Explain the operation of transistorized series voltage regulator with neat circuit diagram and derive the output voltage expression. | CO1 | 15 |
|  | b. | Discuss the short circuit protection mechanism in voltage regulator circuit. | CO1 | 5 |
| 3. | a. | Consider a BJT amplifier which is operating in the voltage divider bias configuration; determine the dc bias voltage IB , IC and VCE. Given VCC = 22V, R1 = 40k, R2 = 3.8k, RC = 10k,RE = 1.5k, CE = 60f, Cin = 40F,  = 140. | CO1 | 15 |
|  | b. | Derive the expression for collector current IC for the collector –base biased BJT with neat circuit diagram. | CO1 | 5 |
| (OR) |  |  |  |  |
| 4. | a. | Derive the expression for base current IB, collector current IC and the collector emitter voltage VCE for a fixed bias BJT network. Also enumerate the procedure to fix the Q point using dc load line analysis. | CO1 | 15 |
|  | b. | Determine the following for the fixed bias FET configuration. i)VGSQ ii)IDSQ. Given VDD=8V, RD=1k, RG=1M, VGG= -2V, IDSS=10mA and VP= -4V. | CO1 | 5 |
| 5. | a. | Discuss the operation and frequency response of RC coupled amplifier with neat circuit diagram. | CO1 | 15 |
|  | b. | Write the functions of the following capacitors in Single stage amplifier. - Bypass Capacitor - Coupling Capacitor - Blocking Capacitor | CO1 | 5 |
| (OR) |  |  |  |  |
| 6. | a. | Prove with necessary statements and equations that the maximum efficiency of class B amplifier is 78.5%. | CO2 | 15 |
|  | b. | Write short notes on class D power amplifier. | CO2 | 5 |
| 7. | a. | Draw the block diagram of Voltage Shunt feedback amplifier and derive its gain, input impedance and output impedance. | CO2 | 15 |
|  | b. | List the features of negative feedback amplifier. | CO2 | 5 |
| (OR) |  |  |  |  |
| 8. | a. | Explain the effect of Current Series feedback on BJT amplifier in terms of the parameters namely input impedance, output impedance and Gain. | CO2 | 15 |
|  | b. | Determine the input impedance of a Current Series feedback amplifier having A=-100 ,Ri=10kΩ , R0=20kΩ and β=-0.5. | CO2 | 5 |
|  |  | Compulsory: |  |  |
| 9. | a. | Obtain the gain, input impedance and output impedance expressions of voltage series feedback amplifier. | CO3 | 15 |
|  | b. | Determine the voltage gain of a voltage series feedback amplifier having A = - 20 and β = -0.2. | CO3 | 5 |