Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14EC2006 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ELECTRONIC CIRCUITS | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Derive the expression of RMS value, efficiency and ripple factor for a full wave rectifier with necessary diagrams. | CO1 | 15 |
|  | b. | Calculate the value of capacitance in a capacitor filter connected to a FWR operating at a aircraft power frequency of 400 HZ, if the ripple factor is 2% for a load of 800Ω. | CO1 | 5 |
| (OR) |  |  |  |  |
| 2. | a. | Explain the operation of transistorized series voltage regulator with neat circuit diagram and derive the output voltage expression. | CO1 | 15 |
|  | b. | Discuss the short circuit protection mechanism in voltage regulator circuit. | CO1 | 5 |
| 3. | a. | Derive the expression for collector current IC and collector-emitter voltage VCE for the voltage divider biased BJT with neat circuit diagrams and necessary steps. | CO1 | 15 |
|  | b. | Predict the Q-point in a fixed bias circuit using BJT with RC = 4 kΩ, RB =1.2MΩ, VCC =10 V, VBE = 0.7 V and β = 80. | CO1 | 5 |
| (OR) |  |  |  |  |
| 4. | a. | Explain the voltage divider bias network for a FET circuit and calculate ID and VGS corresponding to Q-point with necessary diagrams. | CO1 | 15 |
|  | b. | Determine the following for the fixed bias FET configuration. a) VGSQ b) IDSQ. Given VDD=8V, RD=1k, RG=1M, VGG= -2V, IDSS=10mA and VP= -4V. | CO1 | 5 |
| 5. | a. | Elucidate the operation of transformer coupled amplifier with neat circuit diagram. Also write the advantages and disadvantages of this amplifier. | CO1 | 15 |
|  | b. | Write the functions of the following capacitors in Single stage amplifier. - Bypass Capacitor - Coupling Capacitor - Blocking Capacitor | CO1 | 5 |
| (OR) |  |  |  |  |
| 6. | a. | Prove that the maximum efficiency of series fed, directly coupled class A amplifier is 25%. | CO2 | 15 |
|  | b. | Compare Class A and Class B amplifier | CO2 | 5 |
| 7. | a. | Obtain the gain, input impedance and output impedance expressions of voltage series feedback amplifier. | CO2 | 15 |
|  | b. | Determine the voltage gain of a voltage series feedback amplifier having A = - 20 and β = -0.2. | CO2 | 5 |
| (OR) |  |  |  |  |
| 8. | a. | Find the expressions of the gain, input impedance and output impedance in current series feedback amplifier. | CO2 | 15 |
|  | b. | List out the significance of negative feedback amplifier | CO2 | 5 |
|  |  | Compulsory: |  |  |
| 9. | a. | Mention the four basic ways of connecting the feedback signal. Obtain the gain, input and output impedance expressions of voltage shunt feedback amplifier. | CO3 | 15 |
|  | b. | Draw the block diagram of a negative feedback amplifier. | CO3 | 5 |