Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14EC2007 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # TRANSMISSION LINES AND WAVE GUIDES | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Starting from the fundamental, derive the expression for voltage and current at any point on line which is at a distance ‘s’ from the receiving end in terms of receiving end voltage and current. | CO1 | 12 |
|  | b. | A transmission line has the following primary constants measured per km: R=10.15Ω, L=3.93mH, C=0.00797µF, G=0.29 µmho. Determine the Z0 and propagation constant at a frequency of 796 Hz. Also calculate the ratio of current at a point which is 100km down the line to the current at the sending end if the line is terminated in its characteristic impedance. | CO1 | 8 |
| (OR) |  |  |  |  |
| 2. | a. | Explain the characteristics of transmission line not terminated with Z0. Illustrate with necessary phasor diagrams. | CO1 | 8 |
|  | b. | Determine the propagation constant in telephone cable. Discuss, why the distortion effect is severe in telephone cable? | CO1 | 12 |
| 3. | a. | Obtain the expression of voltage and current on a dissipationless line and sketch its phasors for the following load: - open circuit load. - short circuit load. - perfectly matched load. | CO1 | 12 |
|  | b. | Elaborate on standing waves and gives its relation with reflection coefficient. | CO1 | 8 |
| (OR) |  |  |  |  |
| 4. | a. | Build the constant resistance and reactance circle of the smith chart from the impedance relation of the transmission line | CO2 | 14 |
|  | b. | Discuss the impedance matching done using half wave line and gives its significance. | CO2 | 6 |
| 5. | a. | A pair of perfectly conducting planes is separated by 8 cm in air. For frequency of 5000 MHz with the TM10 mode excited, find the following. - Cutoff frequency - Wave impedance - Attenuation constant if operating frequency equals 0.95 times the cutoff frequency. - Phase constant | CO3 | 10 |
|  | b. | Between parallel conducting plates the E field and H field are perpendicular to each other and both of them are perpendicular to the direction of propagation also. Find the following for this electromagnetic wave if air is the dielectric medium between the parallel conducting plates. - Propagation constant, γ - Phase constant, β - Velocity of propagation, vp - Cutoff frequency, fc | CO3 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Sketch the field distribution for TE10 mode in the parallel plate waveguide. | CO3 | 5 |
|  | b. | Explain the characteristics of TE and TM waves. | CO3 | 5 |
|  | c. | Derive the electric and magnetic field components of TE wave propagating between a parallel plate waveguide structure. | CO3 | 10 |
| 7. |  | Derive the electric and magnetic field components from Maxwell’s equation, assuming the wave propagation is along the z-direction in a rectangular waveguide structure. | CO3 | 20 |
| (OR) |  |  |  |  |
| 8. | a. | A X-band waveguide which is air filled has inner dimensions of a = 2.286 cm and b= 1.106 cm. calculate the cut off frequencies of the following modes: TE10,TE20,TE11, TM11, TM21 and TE13. Infer the above modes for signal propagation , when the signal frequency is 10GHz. | CO3 | 10 |
|  | b. | Illustrate different mode excitations in a rectangular waveguide. | CO3 | 10 |
|  |  | Compulsory: |  |  |
| 9. |  | Discuss about the following transmission lines and analyze its significance: - Circular cavity resonator. - Microstrip Lines. - Coplanar Lines. - Slot Lines. | CO3 | 20 |