Reg.No. ____________

End Semester Examination – Apr/May – 2018

| Code : | 14EC2009 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | MICROPROCESSOR AND INTERFACING TECHNIQUES | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Recall the various addressing modes of 8085 with two examples. | CO1 | 15 |
|  | b. | Predict the content of accumulator during the execution of each instruction. LXI H, 5678 H MVI A,48 H ADD H CMP L XRI 0FH RRC | CO1 | 5 |
| (OR) |  |  |  |  |
| 2. | a. | Enumerate the features of 8085 microprocessor with its architecture. | CO1 | 15 |
|  | b. | Interpret the functions of the instruction MOV A,B with the help of timing diagram. Address; Mnemonic; Opcode. 4300; MOV A,B; 78. | CO1 | 5 |
| 3. | a. | Write an ALP in 8085 Microprocessor to find the largest number from an array of 09 numbers. | CO2 | 5 |
|  | b. | Design a memory system for the 8085 microprocessor such that it should contain 1 K RAM, 2K ROM, and 4K EPROM. Give the address mapping. | CO3 | 15 |
| (OR) |  |  |  |  |
| 4. | a. | Interface any one input and output device with 8085 microprocessor. The port address for the input device is AB and port address for the output is 8C. | CO3 | 15 |
|  | b | Write an 8086 assembly language program to transfer a block of 10 data from memory locations 1200 to 1300. | CO2 | 5 |
| 5. | a. | Express the importance of registers and its organization in 8086 microprocessor. | CO2 | 15 |
|  | b. | Predict the output of the registers AX and BX? MOV AX, 0006 SHL AX,02 MOV BX, 00F5 ADD BX,AX INC BL HLT | CO2 | 5 |
| (OR) |  |  |  |  |
| 6. | a. | List down the various addressing modes in 8086 and relate each one with an example. | CO2 | 15 |
|  | b. | Sketch the structure of minimum mode in 8086 microprocessors | CO2 | 5 |
| 7. | a. | Summarise the features of Pentium processors. | CO3 | 8 |
|  | b. | Examine the working of 8253 with different modes of operations. | CO3 | 12 |
| (OR) |  |  |  |  |
| 8. | a. | Predict the control word of 8255 Programmable peripheral interface, when all the ports are connected as output port. Also discuss its functions with a neat block diagram. | CO3 | 10 |
|  | b. | With neat block diagram explain how transmission and reception of data takes place using 8251 serial communication. | CO3 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | Sketch the maximum mode configuration of 8086 microprocessor | CO1 | 6 |
|  | b. | Classify the instruction set of 8085 microprocessor and explain with examples | CO1 | 14 |