Reg.No. ____________

End Semester Examination – Apr/May – 2018

| Code : | 14EC2009 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | MICROPROCESSOR AND INTERFACING TECHNIQUES | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Narrate the features and functions of 8085 microprocessor with a neat architecture. | CO1 | 15 |
|  | b. | Predict the content of accumulator during the execution of each instruction. LXI H, 5678 H MVI A,18 H ADD L CMA ANI F0H RRC | CO1 | 05 |
| (OR) |  |  |  |  |
| 2. | a. | Demonstrate data representation in different addressing Modes of 8085 microprocessor with two examples | CO1 | 10 |
|  | b. | Interpret the functions of the instruction LDA 4500 with the help of timing diagram Address; Mnemonic; Opcode. 4300; LDA 4500; 3A. | CO1 | 10 |
| 3. | a. | Write an ALP in 8085 Microprocessor to add the alternate numbers in an array of 09 numbers | CO1 | 5 |
|  | b. | Design a memory system for the 8085 microprocessor such that it should contain 4 K RAM, 2K ROM, and 8K EPROM. Give the address mapping | CO3 | 15 |
| (OR) |  |  |  |  |
| 4. | a. | Interface a input and output device with 8085 microprocessor. The port address for the input is F2 and port address for the output is A9. . | CO3 | 15 |
|  | b | Calculate the physical address in 8086 microprocessor if the content of segment registers are DS= F219H, SS = 32A8 H, CS= 2000H, ES = 6060 H SP = 7683H, IP = 5921H | CO2 | 05 |
| 5. | a. | Express the importance of registers and its organization in 8086 microprocessor. | CO2 | 15 |
|  | b. | Predict the output of the registers AX and BX? MOV AX, 0006 SHL AX,02 MOV BX, 00F5 ADD BX,AX INC BL HLT | CO2 | 05 |
| (OR) |  |  |  |  |
| 6. | a. | List down the various addressing modes in 8086 and relate each one with an example. | CO2 | 15 |
|  | b. | Sketch the structure of minimum mode in 8086 microprocessors | CO2 | 5 |
| 7. | a. | Describe the working of Programmable peripheral interface, with a neat block diagram. Identify the Mode 0 control word to configure Port A and Port Cu as output ports, port B and port Cl as input ports. | CO3 | 10 |
|  | b. | Examine the working of 8251, receiver and transmitter section. | CO3 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | Recall about on Pentium processors. | CO3 | 08 |
|  | b. | Illustrate the working of keyboard and display interface controller 8279. | CO3 | 12 |
|  |  | Compulsory: |  |  |
| 9. |  | Illustrate the working of programmable timer 8253 with - Block diagram - Different modes of operation - Control word format | CO3 | 20 |