Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14EC2014 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # DIGITAL SIGNAL PROCESSING | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | Determine the output of linear FIR filter whose impulse response is h(n)={1,2,3} and input signal is x(n)={1,2,3,4,5,6,7,8,9} using overlap add method. | CO1 | 20 |
| (OR) |  |  |  |  |
| 2. | a. | Find the circular convolution of two sequences x1(n)={1,2,-3,4,-5} x2(n)={-2,4,6} | CO1 | 15 |
|  | b. | Compare linear and circular convolutions. | CO1 | 5 |
| 3. |  | Calculate the IDFT for the given coefficients X(K)={38, -5.828+j6.07, j6, -0.172+j8.07, -10, -0.172-j8.07, -j6, -5.828-j6.07} using decimation in frequency algorithm | CO1 | 20 |
| (OR) |  |  |  |  |
| 4. |  | Compute the 8 point DFT of a sequence x[n]=2n using radix-2 DIT FFT algorithm. | CO1 | 20 |
| 5. | a. | Design a Chebyshev filter for the following specifications 0.8≤H(ejω)≤1 for 0≤ ω ≤ 0.2π H(ejω)≤0.2 for 0.6π≤ ω ≤ π Use impulse invariant technique. | CO2 | 16 |
|  | b. | Compare one to one mapping and many to one mapping in digital filter design. | CO2 | 4 |
| (OR) |  |  |  |  |
| 6. | a. | Design digital IIR low pass butterworth filter to meet the following specifications: Pass band gain=0.89 Pass band edge frequency=30 Hz Stop band attenuation=0.20 Stop band edge frequency=75 Hz Use Bilinear transformation . Assume sampling frequency is 200 Hz | CO2 | 16 |
|  | b. | Compare FIR and IIR filters | CO2 | 4 |
| 7. |  | The desired response of the filter is Hd(ejω) =e−5jω for π4≤ ¦ω¦≤ 3π40 ; otherwise Determine the frequency response H(ejw) for N=11 using Hanningwindow | CO2 | 20 |
| (OR) |  |  |  |  |
| 8. |  | A LPF has the desired frequency response Hd(ejω)=e−j3ω for 0≤ω ≤ π2 0 for π2< ω ≤ π Find the filter coefficients for N=7 using frequency sampling method | CO2 | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | Explain in detail about finite word length effects in digital filter. | CO3 | 15 |
|  | b. | Compare fixed point and floating point representation. | CO3 | 5 |