Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14EC2014 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # DIGITAL SIGNAL PROCESSING | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | Perform the linear convolution of finite duration sequences h(n)={1,2} and x(n)={1,2,-1,2,3,-2,-3,-1,1,2,-1} by overlap save method. | CO1 | 20 |
| (OR) |  |  |  |  |
| 2. | a. | Find the circular convolution of two sequences x1(n)={1,2,-3,4,-5} x2(n)={-2,4,6} | CO1 | 15 |
|  | b. | Compare linear and circular convolutions. | CO1 | 5 |
| 3. |  | Determine the DFT of the sequence x(n)={1,2,3,4,4,3,2,1}using DIT algorithm. | CO1 | 20 |
| (OR) |  |  |  |  |
| 4. |  | Compute the IDFT of the sequence X(K)={7, -0.707-j0.707, -j, 0.707-j0.707, 1, 0.707+j0.707, j, -0.707+j0.707} using DIF algorithm | CO1 | 20 |
| 5. | a. | Design third order Butterworth digital low pass filter using impulse invariant technique. Assume sampling period T=1 sec. | CO2 | 16 |
|  | b. | Compare FIR and IIR filters. | CO2 | 4 |
| (OR) |  |  |  |  |
| 6. | a. | Determine the transfer function H(z) of the lowest order chebyshev digital filter with the following specification 3 dB ripple in pass band 0≤w≤0.2π 25dB attenuation in stop band 0.45 π ≤w ≤ π Use bilinear transformation. | CO2 | 16 |
|  | b. | Compare one to one mapping and many to one mapping in digital filter design. | CO2 | 4 |
| 7. | a. | The desired response of the filter is Hd(ejω) =e−5jω for π4≤ ¦ω¦≤ 3π40 ; otherwise Determine the frequency response H(ejw) for N=11 using Hanning window. | CO2 | 16 |
|  | b. | Draw the linear phase realization structure for the FIR filter H(z) = ½ + 1/3 z-1 + z -2 + ¼ z-3 + z-4 + 1/3 z-5+ ½ z-6 | CO2 | 4 |
| (OR) |  |  |  |  |
| 8. | a. | Determine the filter coefficientsh(n)obtained by frequency sampling method for N= 7. Hd(ejω) = e−j(N−1)ω2 0 ≤ ω <π20 otherwise | CO2 | 16 |
|  | b. | Define linear phase filter. Derive the condition to be satisfied by the impulse response in order to have a linear phase. | CO2 | 4 |
|  |  | Compulsory: |  |  |
| 9. | a. | Explain in detail about finite word length effects in digital filter. | CO3 | 15 |
|  | b. | Compare fixed point and floating point representation. | CO3 | 5 |