Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14EC2015 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # MICROCONTROLLER AND ITS APPLICATIONS | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. |  | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- | --- |
| 1. | a. |  | Explain the architecture of 8051 Microcontroller with neat block diagram. | CO1 | 15 |
|  | b. |  | Find the contents of the accumulator and flag registers of 8051 after the execution of each instruction. MOV A, #1Ch MOV R5, #0A1h ADD A,R5 ADD A,R5 ADDC A,#10h | CO3 | 5 |
| (OR) |  |  |  |  |  |
| 2. | a. |  | Draw the memory interfacing diagram to connect EPROM 2764 (8K BYTE) and RAM 62256 (32 K BYTE) to the microcontroller 8051. Mention the memory map of each chip. | CO2 | 16 |
|  | b. |  | Debug the following instructions and replace with correct syntax. - MOV A, @DPTR - PUSH R3 - MUL A,B - DEC DPTR | CO3 | 4 |
| 3. | a. |  | Explain the addressing modes of 8051 with examples. | CO1 | 10 |
|  | b. |  | Find the contents of the accumulator of 8051 after the execution of each instruction. MOV A,#0FFh MOV R0,#77h ANL A, R0 MOV 15h, A CPL A ORL 15h,#88h XRL A, 15h XRL A, R0 ANL A, 15h | CO3 | 10 |
| (OR) |  |  |  |  |  |
| 4. |  | a. | Illustrate the different types of jump instructions in 8051 and mention their memory range. | CO2 | 10 |
|  |  | b. | Find the contents of the accumulator and flag registers of 8051 after the execution of each instruction. MOV A,#42h ADD A,#13h DA A ADD A,#17h DA A ADDC A,#34h DA A ADDC A,#11h DA A | CO3 | 10 |
| 5. |  | a. | Analyse how port 0 of 8051 serve as input, output and bidirectional low-order address and data bus for external memory with diagram. | CO2 | 15 |
|  |  | b. | Assume that XTAL = 11.0592 MHz. What value do we need to load the timer’s register if we want to have a time delay of 5 ms. | CO3 | 5 |
| (OR) |  |  |  |  |  |
| 6. | a. |  | Demonstrate any two modes of operations of 8051 Timer. | CO2 | 10 |
|  | b. |  | Write a program for the 8051 to transfer letter “E” serially at 4800 baud rate continuously. | CO3 | 10 |
| 7. |  |  | Draw the architecture of PIC16C74A microcontroller and explain its features in detail. | CO1 | 20 |
| (OR) |  |  |  |  |  |
| 8. | a. |  | Describe the operation of timer 2 mode of PIC with necessary diagrams. Discuss the control register associated with it. | CO2 | 10 |
|  | b. |  | Explain the bit, literal and control instructions of PIC16C74A Microcontroller. | CO2 | 10 |
|  |  |  | Compulsory: |  |  |
| 9. |  |  | Analyse how ADC can be interfaced with a Microcontroller with necessary diagrams. | CO2 | 20 |