Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14EC2021 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # DIGITAL COMMUNICATION | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | With a neat block diagram explain delta modulator and discuss the types of quantization errors occurring in it. Also brief the methods to overcome the shortcomings. | CO1 | 15 |
|  | b. | What is quantization? Explain the types of quantization with necessary diagrams. | CO1 | 5 |
| (OR) |  |  |  |  |
| 2. | a. | Sketch the block diagram of Pulse Code Modulation system and explain the function of each building blocks in detail. | CO1 | 15 |
|  | b. | Using line coding techniques, illustrate any four electrical representation for the data stream "01101001". | CO1 | 5 |
| 3. | a. | Sketch the baseband binary data transmission system. Obtain the Nyquist’s criterion for distortion less baseband binary transmission. | CO1 | 15 |
|  | b. | Sketch the block diagram of Binary Phase Shift Keying transmitter and explain. | CO2 | 5 |
| (OR) |  |  |  |  |
| 4. | a. | Describe the basic operation of matched filter receiver with suitable diagrams. Derive the condition for which the output SNR is maximized. | CO1 | 10 |
|  | b. | Illustrate the DPSK waveform for the input d(t) = 0010011001111 also. Sketch the transmitter and receiver diagram. | CO2 | 10 |
| 5. |  | Sketch the block diagram of MSK system and explain its principle and operation with signal constellation diagram. | CO2 | 20 |
| (OR) |  |  |  |  |
| 6. |  | Sketch the block diagram for the generation and detection of FSK signal. Compare and contrast M-ary PSK and M-ary FSK. | CO2 | 20 |
| 7. |  | A discrete memoryless source has an alphabet of five symbols with their probabilities for its output as given here: Symbol; s0; s1; s2; s3; s4. Probability; 0.55; 0.15; 0.15; 0.10; 0.05. Compute two different Huffman codes for this source. Hence, for each of the two codes, find - The average code-word length. - The variance of the average code-word length over the ensemble of source symbols. - Calculate the efficiency and redundancy. | CO3 | 20 |
| (OR) |  |  |  |  |
| 8. |  | Consider a (6,3) linear block code whose generator matrix is 1 0 0 :1 1 1 G= 0 1 0 :1 1 0 0 0 1 :10 1 - Find all the code vectors and hamming weights. - Determine the error correcting and detecting capability. - Find the parity check matrix H. - Construct the decoding table. | CO3 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | Consider a (6,3) linear block code defined by the generator matrix G= 1 0 0: 1 1 0 0 1 0: 0 1 1 0 0 1: 1 0 1 - Determine if the code is a Hamming code. Find the parity check matrix H of the code in systematic form. - Find the encoding table for the linear block code. - What is the minimum distance of the code? How many errors can the code detect and correct? - Sketch the hardware encoder diagram. - Find the decoding table for the linear block code. - Sketch the hardware syndrome generator - Suppose r=111001 is received, show how the code can correct this error. | CO3 | 20 |