Reg.No. ____________

End Semester Examination – Apr/May – 2018

| Code : | 14EC2027 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | ADVANCED MICROCONTROLLER | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Explain how the 68HC11 microcontroller uses its data/address buses to handle data and address lines. | CO1 | 10 |
|  | b. | Discuss the modes of operation in 68HC11 microcontroller. | CO1 | 8 |
|  | c. | Mention the purpose of power saving mode in a microcontroller | CO1 | 2 |
| (OR) |  |  |  |  |
| 2. | a. | Illustrate the Parallel Input/Output (I/O) Port operation in PIC32 microcontroller. | CO2 | 12 |
|  | b. | Discuss the interrupt system of PIC32 microcontroller. | CO1 | 8 |
| 3. | a. | Write an algorithm and flowchart to interface the blinking of two LEDs controlled by two input switches using interrupt controller of a microcontroller. | CO3 | 10 |
|  | b. | Demonstrate the Traffic light control system using a microcontroller. | CO3 | 10 |
| (OR) |  |  |  |  |
| 4. |  | Apply the internal timer of MC51 and design a RTC with Hr:Min:Sec. Draw the block diagram, flowchart and algorithm for the designed RTC. | CO2 | 20 |
| 5. | a. | Apply the JTAG concept to debug the system. | CO2 | 5 |
|  | b. | Discuss the features of timer in PIC32 microcontroller. | CO2 | 5 |
|  | c. | Explain with a neat diagram the concept of serial communication and the interfacing of RS232 with MC68HC11 | CO3 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Explain internal functions of 68HC11 Timer and its Subsystems. | CO2 | 4 |
|  | b. | Apply SPI protocol in any industrial application and explain its communication in detail with necessary diagrams. | CO3 | 12 |
|  | c. | Discuss the features of Input capture in a microcontroller. | CO2 | 4 |
| 7. |  | Defend the concept of Interfacing CAN Bus With PIC32 Microcontroller For Embedded Networking. | CO3 | 20 |
| (OR) |  |  |  |  |
| 8. | a. | Apply the I/O operation of PIC 32 and control the direct current motor used in any industrial applications in forward and reverse direction. Write the algorithm and explain its interfacing module. | CO2 | 12 |
|  | b. | Pointout how fast the bit manipulation is happening in I/O ports. | CO1 | 8 |
|  |  | Compulsory: |  |  |
| 9. | a. | Design a system to count number of bottles running on a conveyor in a cola industry. Apply the counter module of AVR Atmega microcontroller. Also Explain the internal block diagram of counter module. | CO3 | 16 |
|  | b. | List some Characteristics of AVR microcontroller. | CO1 | 4 |