Reg.No. ____________

End Semester Examination – Apr/May – 2018

| Code : | 14EC2029 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | EMBEDDED SYSTEM DESIGN | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | a. | Outline the characteristics of Embedded systems. | CO1 | 5 |  |  |  |
|  | b. | Design an embedded system which can blink 2 leds based on 2 input switches. Sketch the block diagram and flowchart. | CO1 | 15 |  |  |  |
| (OR) |  |  |  |  |  | neat diagram. |  |
| 2 | a. | Show the difference between : - Microprocessor and Microcontroller. - FPGA and ASIC. - DSP and GPP. | CO1 | 6 |  |  |  |
|  | b. | With a neat diagram elucidate the embedded design life cycle with an example. | CO3 | 14 |  |  |  |
| 3. | a. | Apply the internal timer of 8051 and design a RTC with Hr:Min:Sec. Draw the block diagram, flowchart for the same. | CO2 | 14 |  |  |  |
|  | b. | Illustrate the Hardware and Software issues in designing an Embedded system. | CO2 | 6 |  |  |  |
| (OR) |  |  |  |  |  |  |  |
| 4. | a. | Design an interface diagram of a 12v bulb with 8051 Microcontroller. Write the algorithm to make the bulb glow. | CO3 | 14 |  |  |  |
|  | b. | Write short notes on the following: - Timer. - Counter. - Watchdog Timer. | CO1 | 6 |  |  |  |
| 5. | a. | Sketch the block diagram to Interface RTC with 8051. | CO3 | 5 |  |  |  |
|  | b. | Pinpoint the features of RTC DS12887 | CO3 | 5 |  |  |  |
|  | c. | With a neat diagram Design a biometric attendance system for a class room and write down the algorithm and hardware requirements. | C03 | 10 |  |  |  |
| (OR) |  |  |  |  |  |  |  |
| 6. | a. | Give the block diagram to interface EEPROM with an Embedded controller. | CO3 | 5 |  |  |  |
|  | b. | Explain briefly with a neat block diagram and a flowchart how a DC motor can be interfaced with any microcontroller. Develop an Embedded C program for the same. | CO3 | 15 |  |  |  |
| 7. | a. | Develop an algorithm to generate a time delay for 2 sec for displaying numbers from 0 to 9. | CO3 | 8 |  |  |  |
|  | b. | Design an embedded system for an Automatic Washing machine using RTOS. Explain the different task and give the hardware block diagram. | CO3 | 12 |  |  |  |
| (OR) |  |  |  |  |  |  |  |
| 8. | a. | Write about Deadlock in semaphore. | CO2 | 4 |  |  |  |
|  | b. | Summarise the importance of Scheduler. | CO2 | 4 |  |  |  |
|  | b. | Explain briefly with a neat flow diagram, how semaphore is used between different concurrent tasks. | CO3 | 12 |  |  |  |
|  |  | Compulsory: |  |  |  |  |  |
| 9. | a. | Design an embedded system for an Automatic Chocolate vending machine incorporating RTOS. Explain the different tasks in the system and give the hardware block diagram. | CO2 | 12 |  |  |  |
|  | b. | With a neat diagram explain the life cycle of task and task states | CO3 | 8 |  |  |  |