Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14EC2030 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ARM PROCESSORS | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Write the specifications and features in ARM processor but not in other processor/ controller. | CO1 | 10 |
|  | b. | List out the advantages of RISC over CISC architecture. | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. |  | With a neat diagram explain 3 stage pipeline ARM organization and also elaborate about the modes, registers in ARM. | CO2 | 20 |
| 3. | a. | Explain in detail about ARM instruction set. | CO2 | 12 |
|  | b. | Write a ALP to find the smallest of given array of numbers. | CO2 | 8 |
| (OR) |  |  |  |  |
| 4. |  | Describe the concept of Thumb’s programming model and its related instruction set. | CO1 | 20 |
| 5. | a. | How many bits would the following C datatypes use by the ARM7TDMI? - int - long - char - short - long long | CO1 | 5 |
|  | b. | What will be the output for the following code? MOV R0,#0X11 MOV R1,R0,LSL #1 MOV R2,R1,LSL #1 stop B stop | CO2 | 5 |
|  | c. | Write a program to add 128 bit numbers together, placing the result in registers r0 to r3. The first operand should be placed in registers r4-r7, and second operand should be in r8-r11. | CO2 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Translate the following C code to assembly for (i=0;i<8;i++) { a[i]= b[7-i]; } | CO2 | 10 |
|  | b. | Write the assembly syntax for conditional statements. | CO2 | 10 |
| 7. | a. | What is the significance of direct mapped cache and fully associative cache in ARM? | CO1 | 10 |
|  | b. | Give the significance of context switching in ARM processor. | CO1 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | Explain the importance of memory management unit in ARM. | CO1 | 10 |
|  | b. | ARM supports protection unit. Justify your answer. | CO1 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | Explain how input/ output peripherals are interfaced with ARM core. | CO1 | 10 |
|  | b. | With a neat block diagram explain one biomedical application using ARM processor. | CO3 | 10 |