Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14EC2038 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # CELLULAR MOBILE COMMUNICATION | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Brief about different propagation mechanisms that impact propagation in a mobile communication system. | CO1 | 5 |
|  | b. | Name the modulation techniques used in GSM and Wireless LAN (IEEE 802.11). Bring out in detail the significant features of these modulation techniques. | CO1 | 15 |
| (OR) |  |  |  |  |
| 2. | a. | How the problem of collision is solved in the demand assigned multiple access (DAMA) and multiple access with collision avoidance (MACA) schemes? | CO1 | 5 |
|  | b. | List the advantages of multiplexing in mobile communication. With neat diagrams, explain different multiplexing techniques. | CO1 | 15 |
| 3. |  | Discuss different multiple access techniques used in different generations of cellular communication. Make a comparison table. | CO2 | 20 |
| (OR) |  |  |  |  |
| 4. | a. | If 30MHz of the total bandwidth is allocated for a duplex channels in a cellular wireless system with frequency reuse factor 7 and each simplex channel is 30KHz radio frequency bandwidth, and then find (a) the available number of duplex channels, (b) the number of channels per cell. | CO2 | 5 |
|  | b. | Explain the concept of frequency reuse applied to Cellular Communications with neat diagrams. | CO2 | 15 |
| 5. | a. | Comment on different channel assignment strategies used in cellular communication. | CO2 | 5 |
|  | b. | Kumar is calling his friend Babu from his office landline phone. Babu receives the call in his mobile phone. Give the step-by-step procedure used to setup this call. | CO2 | 15 |
| (OR) |  |  |  |  |
| 6. | a. | In the AMPS system depict the first 84 channels allocated to the cells in the N=7 reuse. Now find the channels allocated to the 5th cell. | CO2 | 5 |
|  | b. | List the different subsystems in GSM and with neat diagrams, explain the working of each subsystem. | CO2 | 15 |
| 7. | a. | Compare and contrast DAB and DVB in terms of bandwidth, mobility support, coverage and cost. | CO2 | 5 |
|  | b. | Give an example of a generic WATM reference model and explain its each block. Also list different WATM services. | CO2 | 15 |
| (OR) |  |  |  |  |
| 8. | a. | Write short notes on geographic position assisted adhoc routing. | CO3 | 5 |
|  | b. | Illustrate the functioning of Destination Sequenced Distance Vector (DSDV) in Mobile Adhoc Network. | CO3 | 15 |
|  |  | Compulsory: |  |  |
| 9. | a. | Mention the functioning of Digital Audio Broadcasting with neat diagrams. | CO2 | 5 |
|  | b. | “Satellite system is the backbone for global mobile communication”- Justify your answer. Also distinguish the satellites in different orbits. | CO2 | 15 |