Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14EC2044 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # FUNDAMENTALS OF WIRELESS COMMUNCATION | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Explain in detail about the technical issues in wireless communication. | CO1 | 12 |
|  | b. | Describe the paging technology and dissect its advantages. | CO1 | 8 |
| (OR) |  |  |  |  |
| 2. | a. | Recall and explain in detail the idea of low power and low cost radios in wireless communication. | CO1 | 8 |
|  | b. | Describe the satellite network and dissect how it withstood market downfall. | CO1 | 12 |
| 3. | a. | This problem demonstrates the capacity increases as cell size decreases. Consider a square city that is 100 square kilometers. Suppose you design a cellular system for this city with square cells, where every cell(regardless of cell size) has 100 channels so can support 100 active users. i.What is the total number of active users that your system can support for a cell size of 1 square kilometer? ii.What cell size would you use if you require that your system support 250,000 active users? | CO2 | 12 |
|  | b. | Describe on the prioritizing techniques for handoff. | CO2 | 8 |
| (OR) |  |  |  |  |
| 4. | a. | Elaborate on the techniques available to improve the coverage and capacity in cellular system. | CO2 | 15 |
|  | b. | Distinguish between adjacent channel interference and co-channel interference. | CO2 | 5 |
| 5. | a. | Deduct the equation of received signal based on free space path loss and add short notes. | CO2 | 14 |
|  | b. | Recall about path loss and shadowing. | CO2 | 6 |
| (OR) |  |  |  |  |
| 6. | a. | # Consider an indoor wireless LAN with fc = 900 MHz, cells of radius 10 m, and nondirectional antennas. Under the free space path loss model, what transmit power is required at the access point in order for all terminals within the cell to receive a minimum power of 10 μW? How does this change if the system Frequency is 5 GHz?. | CO2 | 10 |
|  | b. | Find the simplified multipath delay Tm for a two-ray channel model, when transmitter and receiver separation is relatively large d = 100 m, with ht = 10m and hr = 4m. | CO2 | 10 |
| 7. | a. | Formulate simplified Path loss model. | CO2 | 5 |
|  | b. | Consider a set of empirical measurements of PtPr given in the table below for an indoor system at 900 MHz. Find the path loss exponent γ that minimizes the MSE between the simplified model and the empirical dB power measurements, assuming that d0=1m and K is determined from the free-space path gain formula at this d0. Also find the received power at 150m for the simplified path-loss model with this path-loss exponent and a transmit power of 1mW. Distance from transmitter; M = PrPt. 10 m; -70 dB. 20 m; -75 dB. 50 m; -90 dB. 100 m; -110 dB. 300 m; -125 dB. | CO2 | 15 |
| (OR) |  |  |  |  |
| 8. |  | With proper approximation deduce the time varying impulse response of the channel. | CO3 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | Deduce the expressions and determine the capacity of an AWGN channel. | CO3 | 20 |