Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14EC2065 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # INFORMATION THEORY AND CODING | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Consider the rolling of a dice with the following data, P(1)=P(2)=P(3)=P(4)=P(5)=P(6)=1/6. Determine the information obtained per bit for the outcome of face ‘3’ on rolling the dice. | CO1 | 5 |
|  | b. | If the source generated ‘m’ symbols, then prove that H(X) = log2m | CO1 | 5 |
|  | c. | Determine (i) Entropy of occurrence of rain and snow (ii) Entropy of occurring rain to snow using Markov chain statistical approach. Assume all state probabilities is equal to 1/3. | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Evaluate the probability of occurring to the ‘10’, ‘01’, ‘00’ and ‘11’ given in this state diagram. | CO1 | 8 |
|  | b. | In the message, each letter occurs the following percentage of times: Letter: A B C D E F % of occurrence: 23 20 11 9 15 22 Calculate entropy, average codeword length and efficiency using Shannon-Fano technique. | CO2 | 12 |
| 3. | a. | A discrete memory less source has 5 symbols x1, x2, x3, x4 and x5 with probabilities p(x1)=0.4, p(x2)=0.19, p(x3)=0.16, p(x4)=0.15 and p(x5)=0.1. Calculate entropy, efficiency and code variance by using Huffman coding. | CO2 | 12 |
|  | b. | Distinguish between mutual entropy and mutual information. | CO1 | 4 |
|  | c. | Justify that what would be the maximum entropy, when all the events are equiprobable? | CO1 | 4 |
| (OR) |  |  |  |  |
| 4. | a. | For the 1st order Markov source shown in figure. Draw a tree diagram representing the states at the end of second symbol interval and find the corresponding probabilities. Assume P(1)=P(2)=P(3)=1/3. | CO1 | 15 |
|  | b. | Consider a set of events with probability of occurrence as P(x1)=1/2, P(x2)=1/4, P(x3)=1/8. Compute entropy H(X) and H(X6). | CO1 | 5 |
| 5. | a. | Consider (6,3) linear block code defined by the generating matrix, . Find the parity matrix. | CO3 | 5 |
|  | b. | The mutual information between two discrete random variables X, Yare jointly distributed. Prove that mutual information is I(x,y). | CO2 | 5 |
|  | c. | (i) A Linear block code having a parity check matrix and the message bit is transmitted by the transmitter . Calculate the code vector and syndrome if the received bit is from the transmitter. Show that the error occurred in the received bit is in the second position. | CO3 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Express differential entropy H(x) of a continuous random variable x with probability distribution function. | CO3 | 5 |
|  | b. | Classify the methods used in the error control coding techniques. | CO3 | 5 |
|  | c. | The convolution encoder has a single shift register with 2 stages, constraint length = 3, 3 modulo-2 adders and an output multiplexer. The generated sequence of an encoder are G1=(1,0,1); G2=(1,1,0); G3=(1,1,1). Realize the convolution encoder. | CO3 | 10 |
| 7. | a. | A (7,4) cyclic code with generated polynomial g(x)=1+x+x3, Decode the circuit using shift register. | CO3 | 6 |
|  | b. | Estimate the generator polynomial for the cyclic code. Check if the following codes are cyclic or not. Code X1={0000, 0101, 1010, 1111} Code X2={0000, 0110, 1001, 1111} | CO3 | 4 |
|  | c. | With the following symbol and their probability of occurrence X random variables, encode the message ‘WENT#’ using arithmetic coding algorithm Symbols; E; N; W; T; #. P(X); 0.3; 0.3; 0.1; 0.1; 0.1. | CO3 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | A Gaussian channel has 1MHz bandwidth. Calculate the channel capacity if the signal power to noise spectral density ration S/N is 105Hz. | CO2 | 6 |
|  | b. | Find an expression for an average information content of symbols in long independent sequences. | CO1 | 6 |
|  | c. | Describe in detail about concatenated convolutional code with a neat sketch. | CO3 | 8 |
|  |  | Compulsory: |  |  |
| 9. | a. | Discuss in detail about concatenated convolutional code with a neat sketch. | CO3 | 10 |
|  | b. | Explain briefly on Turbo coding and their applications. | CO3 | 10 |